Applied Physics Lab (LAB-105) 	Semester: Fall 2023
Lab No. 4  (Kirchhoff Voltage Law) 	Max Marks: 10
Date: Week 4

Experiment No. 4

To set up a DC series circuit and verify Kirchhoff Voltage law.

Equipment

Trainer Board

Digital Multimeter (DMM)

Resistors (2.2 K Ω, 1.0 K Ω , 3.9 K Ω)

Theory

Kirchhoff Voltage Law states that sum of voltage rises equals sum of voltage falls, in a closed circuit.

Procedure

- Mount the three resistors in series on the Trainer Board, as shown in the diagram below.
Apply voltage of 5 volts DC from Trainer Board.

- Record voltage readings at points A, B, C and D Table below. Calculate voltage rise and voltage Fall to verify Kirchhoff Voltage Law.

Diagram, Readings and Calculations (On graph side)

2.2 K Ω                                    1.0 K Ω                                     3.9 K Ω

A                                                        B                                                 C                                               D

5V Supply (Trainer)

G

READINGS

| VAG volts | VBG volts | VCG volts | VDG volts |
| --- | --- | --- | --- |

CALCULATIONS

| VAB = VAG – VBG volts | VBC = VBG – VCG volts | VCD = VCG – VDG volts |
| --- | --- | --- |

| Total Voltage Rise VAG volts | Total Voltage Fall VAB + VBC + VCD volts |
| --- | --- |