Applied Physics Lab (LAB-105) 	Semester: Fall 2023
Lab No. 6  (Amplitude and Freq. measurement) 	Max Marks: 10
Date: Week 6

Experiment No. 6

To measure amplitude and frequency using Cathode Ray Oscilloscope (CRO) and Function Generator.

Equipment

Cathode Ray Oscilloscope (CRO)

Function Generator

Procedure

- Set the output of Function generator to frequency of 10 KHz with sine wave output.

- Connect output of Function generator to input of CRO.

- Observe the waveform and measure its amplitude and time period using CRO controls. Calculate its amplitude and frequency and compare it with Function generator output.

- Repeat the above steps for Square wave of 15 KHz and Triangular wave of 20 KHz.

- Draw all three waveforms in your notebook.

Readings (on Graph Sheet)

Sine Wave ( 10 kHz)

Output obtained from Function generator: Amplitude  ____ Volt ,    Frequency = ____ Hz

Display on CRO:

Amplitude Setting V/Div= ___    Reading =  ____  Div     Amplitude Calculated =  _____ volts
Time base setting Time/Div = ___ Reading =  ___Div

Time period (Calculated) =  _____ sec      Frequency (Calculated) =  ___ Hz

Square Wave ( 15 kHz)

Output obtained from Function generator: Amplitude  ____ Volt ,    Frequency = ____ Hz

Display on CRO:

Amplitude Setting V/Div= ___    Reading =  ____  Div     Amplitude Calculated =  _____ volts
Time base setting Time/Div = ___ Reading =  ___Div

Time period (Calculated) =  _____ sec      Frequency (Calculated) =  ___ Hz

Triangular Wave ( 20 kHz)

Output obtained from Function generator: Amplitude  ____ Volt ,    Frequency = ____ Hz

Display on CRO:

Amplitude Setting V/Div= ___    Reading =  ____  Div     Amplitude Calculated =  _____ volts
Time base setting Time/Div = ___ Reading =  ___Div

Time period (Calculated) =  _____ sec      Frequency (Calculated) =  ___ Hz