Applied Physics Lab (LAB-105) 	Semester: Fall 2023
Lab No. 7  (p-n junction diode) 	Max Marks: 10
Date: Week 7

Experiment No. 7

To check the operation of a p-n junction in forward and reverse bias.

Equipment

Trainer Board, Var. Power Supply, Digital Multimeter (2), P-N junction Diode (HER107), Resistor 2.2 KOhm

Theory

The p-n junction diode allows current to flow in forward bias and does not allow current to flow in reverse bias. This gives the diode the property of a unidirectional device and can function as an ON-OFF switch.

Procedure

- Mount the p-n junction on breadboard and connect it in forward bias, as shown in Fig 1.

- Change the applied voltage from the variable power supply in steps. In each step, measure the voltage across the diode VF and the current through it IF. Record your readings in table (a).

- Now reverse the position of the diode so that it is in reverse biased mode. Change the applied voltage VR in steps and note the corresponding reverse current IR. Record your readings in table (b).

- From your readings in 2 and 3 above, draw the forward and reverse characteristics of the diode.

Readings (on Graph Paper)

- Diode in Forward Bias

| Sr. No. | VF from Circuit (Volts) | IF from Circuit (mA) |
| --- | --- | --- |

- Diode in Reverse Bias

| Sr. No. | VR from Circuit (Volts) | IR from Circuit (mA) |
| --- | --- | --- |

R= 2.2 K                                                                           R= 2.2 K

VF (DMM-1)                                                                VR (DMM-1)

+                                                                                               +

PS                                                                                      PS

-                IF                                                                    -                IR

DMM-2                                                                                        DMM-2

Fig: 1							Fig: 2