Applied Physics Lab (LAB-105) 	Semester: Fall 2023
Lab No. 8  (Bipolar Junction Transistor) 	Max Marks: 10
Date: Week 8

Experiment No. 8

To study the operation of a Bipolar Junction Transistor (BJT) as a switch.

Equipment

Trainer Board
Lab Power Supply 
Digital Multimeter (x 2)
Transistor (NPN)  2N3904
Resistors 2.2 K Ohm  and 7.5 K Ohm

Theory

The N-P-N transistor (2N 3904) is connected in Common Emitter Configuration, i.e. Emitter is common to the input and output circuits. The input circuit is Base-Emitter and the output circuit is Collector-Emitter. In this experiment the input voltage (VBE) is changed and effect on output voltage (VCE) is observed. The transistor will move from Cut-off state (No current in Output circuit) to Saturation state (Maximum current in Output circuit). A small change in VBE (and IB) results in corresponding large change in VCE (and IC). This is called Transistor Action.

Procedure

- Mount the transistor on the breadboard after checking its pins (Emitter, Base, Collector) and connect the circuit as shown in the diagram. Vary VBB from the Variable Power supply.

- Note down the readings of VBE and VCE, and calculate IB and IC.

- Mention if transistor is in Cut-off or Saturation state (in Remarks Column).

Circuit and Readings (on Graph Paper)

RC = 2.2K

IC

RB = 7.5K

VCC = 5V

VBB

(variable)

| Sr. No. | VBE (volts) | VCE (volts) | IB (mA) Calculate using Kirchhoff Law | IC (mA) Calculate using Kirchhoff Law | Remarks Cut-off or Saturation |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |