BEACONHOUSE NATIONAL UNIVERSITY School of Computer & Information Technology LAB ASSIGNMENT 3 DNS & Web Server Configuration Built from Scratch — Cisco Packet Tracer

| Course | Computer Networks Lab (Lab-209) |
| --- | --- |
| Semester | Spring 2026 |
| Institution | Beaconhouse National University |
| Instructor | Shazia Rizwan |
| Lab Instructor | Hasnain Ahmad |
| Total Marks | 20 Marks |
| CLOs Covered | CLO 4 (Apply — C3) |
| Submission | Submit .pkt file + screenshots via LMS |

Student Name: ________________________________     Roll No: __________________     Section: ________

# 1. Learning Objectives

By the end of this lab, you will be able to:

- Build a multi-device network topology from scratch in Cisco Packet Tracer

- Configure a Server-PT as a combined DNS and Web server for youtube.com

- Set up an INTERNET router to interconnect the server and the campus network

- Configure R1 with interface IPs, a default route, a DNS server entry, and local ip host records

- Set PC1, PC2, and PC3 to use the external DNS server for hostname resolution

- Ping remote devices by hostname instead of IP from both R1 and the PCs

- Use Simulation Mode to inspect DNS Query and DNS Reply packets step by step

📝 Mapped CLO CLO 4 — Configure network services in Cisco Packet Tracer [Apply — Bloom's C3]

# 2. Background

Server-PT at 1.1.1.1 performs two roles simultaneously: it is the DNS server that resolves youtube.com, and it is the web server that serves the youtube.com web page. When a PC types youtube.com in a browser, it first queries 1.1.1.1 for the IP address, then contacts that IP to load the page — all within the same server.

| Concept | Explanation |
| --- | --- |
| DNS Server | Stores hostname-to-IP mappings (A records). When queried with 'youtube.com', it returns the IP of the server hosting that site. |
| A Record | A DNS record mapping a hostname to an IPv4 address. In this lab: youtube.com → 1.1.1.1 (the same server hosts both DNS and the web page). |
| Web Server (HTTP) | Runs HTTP service on port 80. Serves web pages when a client sends an HTTP GET request. Server-PT has a built-in HTTP service. |
| Default Route | A catch-all route (0.0.0.0/0) on R1. Any packet destined for the internet (e.g. 1.1.1.1) is forwarded via this route toward the INTERNET router. |
| ip name-server | Cisco IOS command that sets which DNS server the router itself uses. R1 will query 1.1.1.1 for names it cannot resolve locally. |
| ip host | Cisco IOS command that creates a static local hostname-to-IP mapping on the router — like a local hosts file. Used for R1, PC1, PC2, PC3. |
| Simulation Mode | Pauses Packet Tracer's real-time mode so you can step through each packet and inspect DNS query/reply contents in detail. |

# 3. Lab Topology

The topology below shows exactly what you will build. Study every interface label and IP address before starting.

📸 Topology Diagram Reference Study the topology carefully before placing any devices. Key points: Server-PT is on the 1.1.1.0/24 network, the WAN link between INTERNET and R1 is 203.0.113.0/30, and R1's LAN interface connects to SW1 at 192.168.0.254. The INTERNET router takes .2 and R1 takes .1 on the WAN link — note this is the reverse of a typical setup.

## 3.1  IP Address Plan

| Device | Interface | IP Address | Subnet Mask | Notes |
| --- | --- | --- | --- | --- |
| Server-PT (youtube.com / DNS) | NIC | 1.1.1.1 | 255.255.255.0 | DNS + Web server — same device |
| INTERNET Router | G0/0 | 203.0.113.2 | 255.255.255.252 | Faces R1 on /30 WAN link (.2 side) |
| INTERNET Router | G0/1 | 1.1.1.254 | 255.255.255.0 | Faces Server-PT |
| R1 | G0/0 | 203.0.113.1 | 255.255.255.252 | Faces INTERNET Router (.1 side) |
| R1 | G0/1 | 192.168.0.254 | 255.255.255.0 | LAN gateway for PCs |
| SW1 | — | — | — | Layer-2 switch, no IP needed |
| PC1 | NIC | 192.168.0.1 | 255.255.255.0 | GW: 192.168.0.254 DNS: 1.1.1.1 |
| PC2 | NIC | 192.168.0.2 | 255.255.255.0 | GW: 192.168.0.254 DNS: 1.1.1.1 |
| PC3 | NIC | 192.168.0.3 | 255.255.255.0 | GW: 192.168.0.254 DNS: 1.1.1.1 |

⚠️ Important — WAN Link Direction The WAN link between INTERNET and R1 is 203.0.113.0/30. INTERNET router = .2 ¦ R1 = .1 — this is the REVERSE of a typical setup. Follow the diagram exactly. Do not mix these up or routing will fail.

## 3.2  Devices to Place in Packet Tracer

| Device Type | Model | Qty | Label in Workspace |
| --- | --- | --- | --- |
| Router | 2911 | 2 | INTERNET, R1 |
| Switch | 2960-24TT | 1 | SW1 |
| Server | Server-PT | 1 | Server-PT (labelled: youtube.com / 1.1.1.1) |
| PC | PC-PT | 3 | PC1, PC2, PC3 |

## 3.3  Cable Connections

| From | Port | To | Port | Cable Type |
| --- | --- | --- | --- | --- |
| Server-PT | FastEthernet0 | INTERNET Router | GigabitEthernet0/1 | Straight-Through |
| INTERNET Router | GigabitEthernet0/0 | R1 | GigabitEthernet0/0 | Straight-Through |
| R1 | GigabitEthernet0/1 | SW1 | GigabitEthernet0/1 | Straight-Through |
| SW1 | FastEthernet0/1 | PC1 | FastEthernet0 | Straight-Through |
| SW1 | FastEthernet0/2 | PC2 | FastEthernet0 | Straight-Through |
| SW1 | FastEthernet0/3 | PC3 | FastEthernet0 | Straight-Through |

💡 Tip — Wait for Green Link Lights After placing all cables, wait a few seconds for orange link lights to turn green. If a link stays orange, double-click the cable and verify the correct port was selected on each device. All lights MUST be green before beginning configuration.

# 4. Building the Network from Scratch

Configure every device in the order given below. Each device must be fully set up before moving to the next.

DEVICE 1 — Configure Server-PT DNS Server + Web Server ¦ IP: 1.1.1.1

Server-PT performs two roles simultaneously: it is the DNS server that holds the youtube.com A record, and it is the HTTP web server that delivers the youtube.com web page. Its IP (1.1.1.1) is the address that PCs and R1 will use as their DNS server.

## Step 1.1 — Assign Static IP

- Click on Server-PT to open it.

- Go to Desktop → IP Configuration → select Static.

- Enter the following values:

| Field | Value |
| --- | --- |
| IP Address | 1.1.1.1 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 1.1.1.254 |
| DNS Server | 1.1.1.1 (the server points to itself) |

📸 SCREENSHOT REQUIRED — Server-PT IP Configuration Take a screenshot of Server-PT → Desktop → IP Configuration showing: • IP Address: 1.1.1.1 • Subnet Mask: 255.255.255.0 • Default Gateway: 1.1.1.254 • DNS Server: 1.1.1.1 Label it: 'Screenshot 1 — Server-PT IP Configuration'

## Step 1.2 — Enable DNS Service and Add A Record

- Click the Services tab → click DNS in the left panel.

- Toggle DNS Service to ON.

- Fill in the record fields and click Add:

| Field | Value |
| --- | --- |
| Name | youtube.com |
| Type | A Record |
| Address | 1.1.1.1 (Server-PT's own IP — it hosts the web page too) |

✅ Expected Result — DNS A Record Added After clicking Add, you should see youtube.com listed in the DNS records table with address 1.1.1.1. This means: when any client asks 'where is youtube.com?', the server answers '1.1.1.1'.

📸 SCREENSHOT REQUIRED — Server-PT DNS Tab Take a screenshot of Server-PT → Services → DNS showing: • DNS Service: ON • Record: youtube.com ¦ A Record ¦ 1.1.1.1 visible in the table Label it: 'Screenshot 2 — Server-PT DNS Record'

## Step 1.3 — Enable HTTP Web Service and Customise the Page

- Still on Server-PT → Services tab → click HTTP in the left panel.

- Confirm HTTP is ON.

- Click the edit button next to index.html and replace the page content:

<html>

<body>

<h1>Welcome to YouTube - BNU SCIT Lab Simulation</h1>

<p>You have successfully resolved youtube.com via DNS.</p>

</body>

</html>

- Click Save after editing.

✅ Expected Result — HTTP + DNS on Same Server With both DNS and HTTP enabled on the same server, a PC that resolves youtube.com gets 1.1.1.1 back, then sends an HTTP request to 1.1.1.1, and receives this customised web page.

📸 SCREENSHOT REQUIRED — Server-PT HTTP Tab Take a screenshot of Server-PT → Services → HTTP showing: • HTTP Service: ON • The edited index.html content visible in the text area Label it: 'Screenshot 3 — Server-PT HTTP Custom Page'

DEVICE 2 — Configure the INTERNET Router 2911 Router ¦ Connects Server-PT to R1

The INTERNET router connects Server-PT to R1. It has two interfaces: G0/1 faces Server-PT on the 1.1.1.0/24 network, and G0/0 faces R1 on the 203.0.113.0/30 WAN link. It also needs a static route back to the campus LAN so that reply packets can reach the PCs.

## Step 2.1 — Configure Interfaces

Open the INTERNET router CLI (click the router → CLI tab) and enter the following:

! Open INTERNET router CLI and enter config mode

Router> enable

Router# configure terminal

Router(config)# hostname INTERNET

! G0/1 faces Server-PT (1.1.1.0/24 network)

INTERNET(config)# interface GigabitEthernet0/1

INTERNET(config-if)# ip address 1.1.1.254 255.255.255.0

INTERNET(config-if)# no shutdown

INTERNET(config-if)# exit

! G0/0 faces R1 on the 203.0.113.0/30 WAN link (.2 side per topology)

INTERNET(config)# interface GigabitEthernet0/0

INTERNET(config-if)# ip address 203.0.113.2 255.255.255.252

INTERNET(config-if)# no shutdown

INTERNET(config-if)# exit

! Static route back to campus LAN so DNS replies can reach the PCs

INTERNET(config)# ip route 192.168.0.0 255.255.255.0 203.0.113.1

INTERNET(config)# end

INTERNET# write memory

## Step 2.2 — Verify INTERNET Router

! Confirm both interfaces are up/up

INTERNET# show ip interface brief

! Should show: 2 connected routes + 1 static route to 192.168.0.0/24

INTERNET# show ip route

! Ping Server-PT to confirm G0/1 side is working

INTERNET# ping 1.1.1.1

✅ Expected Result — INTERNET Router Verified Both GigabitEthernet interfaces should appear as 'up/up' with their correct IPs. G0/0: 203.0.113.2 ¦ G0/1: 1.1.1.254 The ping to 1.1.1.1 must succeed (5 exclamation marks) before proceeding. show ip route should show C 1.1.1.0/24, C 203.0.113.0/30, and S 192.168.0.0/24.

📸 SCREENSHOT REQUIRED — INTERNET Router: show ip interface brief Take a screenshot of INTERNET# show ip interface brief showing both G0/0 and G0/1 as 'up/up'. Label it: 'Screenshot 4 — INTERNET Router show ip interface brief'

📸 SCREENSHOT REQUIRED — INTERNET Router: show ip route Take a screenshot of INTERNET# show ip route showing connected + static routes. Label it: 'Screenshot 5 — INTERNET Router show ip route'

DEVICE 3 — Configure R1 Campus Router ¦ Most Critical Device

R1 is the most important device to configure correctly. It connects the campus LAN to the internet side. It also acts as a local DNS resolver — holding static host entries for R1, PC1, PC2, and PC3 — and forwards any other DNS queries upstream to 1.1.1.1.

## Step 3.1 — Configure Interfaces

! Enter R1 CLI

Router> enable

Router# configure terminal

Router(config)# hostname R1

! G0/0 — WAN interface toward INTERNET router (.1 side of 203.0.113.0/30)

R1(config)# interface GigabitEthernet0/0

R1(config-if)# ip address 203.0.113.1 255.255.255.252

R1(config-if)# no shutdown

R1(config-if)# exit

! G0/1 — LAN interface — default gateway for PC1, PC2, PC3

R1(config)# interface GigabitEthernet0/1

R1(config-if)# ip address 192.168.0.254 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

## Step 3.2 — Configure Default Route

! Send all unknown traffic (internet-bound) toward INTERNET router

! Next hop = 203.0.113.2 (INTERNET router's G0/0 IP)

R1(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.2

⚠️ Warning — Correct Next-Hop for Default Route The next-hop IP for R1's default route is 203.0.113.2 (INTERNET router's G0/0 address). Do NOT use 203.0.113.1 — that is R1's own G0/0 address. Incorrect next-hop = no internet connectivity.

## Step 3.3 — Configure DNS Server on R1

! Tell R1 which DNS server to use when it resolves names itself

R1(config)# ip name-server 1.1.1.1

! Ensure DNS lookups are enabled on R1 (sometimes disabled by default)

R1(config)# ip domain-lookup

## Step 3.4 — Create Local Host Entries on R1

Host entries (ip host) create a local hostname-to-IP table on R1. When R1 or anything using R1 as a DNS resolver asks for PC1, PC2, PC3, or R1 by name, the router answers immediately from this local table without querying 1.1.1.1.

! Local A record for R1 itself (WAN IP and LAN IP both registered)

R1(config)# ip host R1 203.0.113.1 192.168.0.254

! Local A record for PC1

R1(config)# ip host PC1 192.168.0.1

! Local A record for PC2

R1(config)# ip host PC2 192.168.0.2

! Local A record for PC3

R1(config)# ip host PC3 192.168.0.3

R1(config)# end

R1# write memory

## Step 3.5 — Verify R1 Full Configuration

! Check all interfaces are up/up with correct IPs

R1# show ip interface brief

! Routing table: should show S* default + 2 directly connected routes

R1# show ip route

! View all local host entries

R1# show hosts

! Ping INTERNET router WAN IP — tests default route

R1# ping 203.0.113.2

! Ping Server-PT — tests full routing through INTERNET router

R1# ping 1.1.1.1

! Ping PC1, PC2, PC3 by hostname — tests local ip host entries

R1# ping PC1

R1# ping PC2

R1# ping PC3

! Resolve youtube.com from R1 — tests DNS query to 1.1.1.1

R1# ping youtube.com

✅ Expected Result — R1 Fully Verified show ip interface brief: G0/0 (203.0.113.1) and G0/1 (192.168.0.254) both up/up. show ip route: S* 0.0.0.0/0 [1/0] via 203.0.113.2 plus two C (connected) routes. show hosts: R1, PC1, PC2, PC3 all listed as perm (permanent). All pings to 203.0.113.2, 1.1.1.1, PC1, PC2, PC3, and youtube.com must succeed. ping youtube.com from R1 confirms end-to-end DNS and routing.

📸 SCREENSHOT REQUIRED — R1: show ip route Take a screenshot of R1# show ip route. Must show: S* 0.0.0.0/0 [1/0] via 203.0.113.2 and two C (connected) routes. Label it: 'Screenshot 6 — R1 show ip route'

📸 SCREENSHOT REQUIRED — R1: show hosts Take a screenshot of R1# show hosts. Must show all 4 permanent entries: R1, PC1, PC2, PC3. Label it: 'Screenshot 7 — R1 show hosts'

DEVICE 4 — Configure PC1, PC2, and PC3 Static IP + DNS = 1.1.1.1

Each PC needs a static IP address, the correct default gateway (R1's G0/1), and DNS server set to 1.1.1.1 (Server-PT). Without the correct DNS entry, pinging or browsing by hostname will fail even if routing works perfectly.

## Step 4.1 — Configure Each PC

- Click on PC1 → Desktop tab → IP Configuration → select Static.

- Enter the values for PC1 from the table below.

- Repeat for PC2 and PC3 with their respective IPs.

| PC | IP Address | Subnet Mask | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- |
| PC1 | 192.168.0.1 | 255.255.255.0 | 192.168.0.254 | 1.1.1.1 |
| PC2 | 192.168.0.2 | 255.255.255.0 | 192.168.0.254 | 1.1.1.1 |
| PC3 | 192.168.0.3 | 255.255.255.0 | 192.168.0.254 | 1.1.1.1 |

💡 Tip — Configuring Multiple PCs Quickly Configure all three PCs before moving to the task section. For PC2 and PC3, only change the last octet of the IP address (from .1 to .2 and .3). All other settings — subnet mask, gateway, DNS — remain identical.

📸 SCREENSHOT REQUIRED — PC1, PC2, PC3 IP Configuration Take a screenshot of each PC's Desktop → IP Configuration showing DNS Server = 1.1.1.1. Label them: 'Screenshot 8a — PC1 IP Config', 'Screenshot 8b — PC2 IP Config', 'Screenshot 8c — PC3 IP Config'

# 5. Lab Tasks

Now that all devices are configured, execute the following 4 tasks in order. Each task corresponds directly to the lab assignment brief.

TASK 1 — Verify Default Route to the Internet on R1 CLO 4 — Apply (C3)

The default route was already configured in Step 3.2. This task verifies it is correct and that it enables traffic from R1 to reach Server-PT on the internet side.

! On R1 — verify the default route exists in the routing table

R1# show ip route

! Look for this line in the output:

S* 0.0.0.0/0 [1/0] via 203.0.113.2

! Verify end-to-end reachability from R1 to Server-PT

R1# ping 1.1.1.1

! Expected output:

!!!!! Success rate is 100 percent (5/5)

✅ Expected Result — Default Route Confirmed The S* line confirms the default route is in place. Five exclamation marks (!!!!! ) confirm R1 can reach Server-PT across the INTERNET router.

⚠️ Troubleshooting — If Ping Fails 1. Check both INTERNET router interfaces are up/up (show ip interface brief on INTERNET). 2. Verify static route on INTERNET router: ip route 192.168.0.0 255.255.255.0 203.0.113.1 3. Confirm R1's G0/0 IP is 203.0.113.1 (not .2). 4. Confirm INTERNET router G0/0 is 203.0.113.2 (not .1).

📸 SCREENSHOT REQUIRED — R1: show ip route (Task 1) Take a screenshot showing the S* 0.0.0.0/0 line and the ping 1.1.1.1 output (!!!!!) Label it: 'Screenshot 9 — Task 1: R1 Default Route and Ping'

📝 Task 1 Questions — Answered Q1: Copy the full S* line from 'show ip route' on R1: S* Entry: S* 0.0.0.0/0 [1/0] via 203.0.113.2 Q2: What does the [1/0] value mean in the routing table output? Answer: [1/0] represents [Administrative Distance / Metric]. Administrative Distance (AD) = 1 means it is a static route (AD of 1 is the default for static routes, making them highly preferred over dynamic routes). Metric = 0 means there is no hop-count or cost-based metric associated — the router simply follows the next-hop IP address 203.0.113.2 without any additional calculation.

TASK 2 — Verify PC1, PC2, PC3 Use 1.1.1.1 as DNS and Can Ping by Name CLO 4 — Apply (C3)

DNS was set on each PC in Step 4.1. Now verify it is working by confirming the settings and testing name resolution from the PCs.

## Verify DNS is Configured on Each PC

- Click on PC1 → Desktop → IP Configuration.

- Confirm DNS Server shows 1.1.1.1. Take a screenshot.

- Repeat for PC2 and PC3.

## Test Name Resolution from PC1

Click PC1 → Desktop → Command Prompt. Run the following commands:

! Confirm PC1's full IP configuration

C:\> ipconfig

! Ping the default gateway (R1) by IP — confirms basic LAN connectivity

C:\> ping 192.168.0.254

! Ping Server-PT by IP — confirms routing to internet side works

C:\> ping 1.1.1.1

! Ping youtube.com by NAME — this is the key DNS test

! PC1 will query 1.1.1.1 first, get back 1.1.1.1, then ping it

C:\> ping youtube.com

✅ Expected Result — DNS Name Resolution Working The 'ping youtube.com' output should show: Pinging youtube.com [1.1.1.1] The brackets [1.1.1.1] confirm DNS resolved the name to the correct IP. Then 4 successful replies (Reply from 1.1.1.1: bytes=32 time<1ms TTL=126). Repeat the same ping youtube.com from PC2 and PC3 — all should succeed.

🔧 Try It Yourself — Web Browser Test Open a web browser on PC1: Desktop → Web Browser. Type http://youtube.com in the URL bar and press Go. The custom HTML page you configured on Server-PT should load. This confirms the full DNS → HTTP flow is working end to end.

📸 SCREENSHOT REQUIRED — PC1: All Pings Take a screenshot of PC1's Command Prompt showing pings to: 192.168.0.254, 1.1.1.1, and youtube.com. Label it: 'Screenshot 10 — PC1 All Ping Tests'

📸 SCREENSHOT REQUIRED — PC1: Web Browser Take a screenshot of PC1's Desktop → Web Browser showing http://youtube.com loaded. The custom HTML page 'Welcome to YouTube - BNU SCIT Lab Simulation' must be visible. Label it: 'Screenshot 11 — PC1 Web Browser HTTP Test'

📝 Task 2 Questions — Answered Q1: What IP address was returned when PC1 pinged youtube.com by name? Resolved IP: 1.1.1.1 Q2: Did all three PCs successfully resolve and ping youtube.com? PC1: Yes — Pinging youtube.com [1.1.1.1] — 4 replies received PC2: Yes — Pinging youtube.com [1.1.1.1] — 4 replies received PC3: Yes — Pinging youtube.com [1.1.1.1] — 4 replies received Q3: Did the web browser on PC1 load the youtube.com page? Describe what you saw: Answer: Yes. The browser on PC1 loaded the custom HTML page configured on Server-PT. The page displayed: 'Welcome to YouTube - BNU SCIT Lab Simulation' as a large heading, followed by the paragraph 'You have successfully resolved youtube.com via DNS.' This confirmed that PC1 sent an HTTP GET request to 1.1.1.1 (after DNS resolved youtube.com to that IP), and Server-PT responded with the custom index.html page.

TASK 3 — Verify R1 Uses 1.1.1.1 as DNS and Host Entries Work CLO 4 — Apply (C3)

The ip name-server and ip host entries were configured in Device 3. This task verifies that they are correct and that R1 can resolve both local hostnames (from its own ip host table) and external names (via DNS query to 1.1.1.1).

## Verify the Host Table on R1

! Show all static host entries on R1

R1# show hosts

! Expected output — four entries all marked as 'perm' (permanent):

Host Port Flags Age Type Address(es)

R1 None (perm, OK) 0 IP 203.0.113.1 192.168.0.254

PC1 None (perm, OK) 0 IP 192.168.0.1

PC2 None (perm, OK) 0 IP 192.168.0.2

PC3 None (perm, OK) 0 IP 192.168.0.3

## Ping Hosts by Name from R1

! Ping PC1 by hostname — R1 looks up its local ip host table

R1# ping PC1

! Ping PC2 and PC3 by hostname

R1# ping PC2

R1# ping PC3

! Also verify R1 can resolve external names via 1.1.1.1

R1# ping youtube.com

✅ Expected Result — All Pings Succeed from R1 All four pings should return 5 successful replies (!!!!! for each). 'ping PC1' confirms the local ip host entry is working — no DNS query sent. 'ping youtube.com' confirms R1's DNS query to 1.1.1.1 returns 1.1.1.1 and the server is reachable.

⚠️ Troubleshooting — If ping PC1 Fails Check the ip host entry was entered correctly: R1(config)# ip host PC1 192.168.0.1 If ping youtube.com fails: run 'show run ¦ include name-server' — must show ip name-server 1.1.1.1 Also verify 1.1.1.1 is reachable: R1# ping 1.1.1.1

📸 SCREENSHOT REQUIRED — R1: ping PC1 by Name Take a screenshot of R1# ping PC1 showing 5 successful replies. Label it: 'Screenshot 12 — R1 ping PC1 by hostname'

📝 Task 3 Questions — Answered Q1: Run 'show hosts' on R1. List all four host entries exactly as they appear: 1. R1 → 203.0.113.1 192.168.0.254 (perm, OK) 2. PC1 → 192.168.0.1 (perm, OK) 3. PC2 → 192.168.0.2 (perm, OK) 4. PC3 → 192.168.0.3 (perm, OK) Q2: When R1 pings 'PC1' by name, does it send a DNS query to 1.1.1.1 first, or use its local ip host table? How do you know? Answer: R1 uses its local ip host table — NOT a DNS query to 1.1.1.1. Evidence: The 'show hosts' command shows PC1 listed with the flag 'perm' (permanent). This means the entry was statically configured via the 'ip host' command and is resolved locally. In Simulation Mode, no DNS UDP packet (port 53) would be generated when pinging PC1 — the router resolves the name instantly from its own table without querying any external server. Cisco IOS always checks its local ip host table before sending a DNS query.

TASK 4 — Ping youtube.com from PC1 and Analyse in Simulation Mode CLO 4 — Apply (C3) ¦ Most Critical Task

This is the most critical task. You will switch Packet Tracer to Simulation Mode and watch every packet involved when PC1 types 'ping youtube.com' — from the DNS query leaving PC1, to the DNS reply arriving back, to the ICMP ping reaching Server-PT.

## Part A — Ping youtube.com by Name from PC1

- Click on PC1 → Desktop → Command Prompt.

- Type the following and press Enter:

! Ping youtube.com by name — this triggers a DNS lookup first

C:\> ping youtube.com

! Expected output:

Pinging youtube.com [1.1.1.1] with 32 bytes of data:

Reply from 1.1.1.1: bytes=32 time<1ms TTL=126

Reply from 1.1.1.1: bytes=32 time<1ms TTL=126

Reply from 1.1.1.1: bytes=32 time<1ms TTL=126

Reply from 1.1.1.1: bytes=32 time<1ms TTL=126

✅ Expected Result — DNS + Ping Confirmed The brackets [1.1.1.1] after youtube.com confirm that DNS successfully resolved the name. The 4 replies confirm end-to-end reachability through two routers (R1 and INTERNET).

## Part B — Observe DNS Messages in Simulation Mode

🔬 Simulation Mode — Step-by-Step Instructions - In the bottom-right corner of Packet Tracer, click the Simulation tab (or press Shift+S). - In the Event List Filters section, click 'Show All / None' to clear everything. - Tick only these protocols: DNS ¦ ICMP - Click on PC1 → Desktop → Command Prompt. - Type: ping youtube.com and press Enter. - Click the 'Capture / Forward' (▶¦) button one step at a time. - Click on each coloured envelope that appears on the topology. - In the PDU Information window, look at the Outbound PDU Details. - Identify the DNS Query and DNS Reply packets by their protocol label. - Record what you see in the Task 4 Question box below.

## What to Look for in Each Packet

| Step | Protocol | Direction | What You Should See |
| --- | --- | --- | --- |
| 1 | DNS (UDP 53) | PC1 → Server-PT | DNS Query: Source = 192.168.0.1, Dest = 1.1.1.1, Port 53, Question = 'youtube.com' |
| 2 | DNS (UDP 53) | Server-PT → PC1 | DNS Reply: Answer section shows youtube.com A record → 1.1.1.1 |
| 3 | ICMP | PC1 → Server-PT | ICMP Echo Request: Dest = 1.1.1.1 (resolved IP from DNS) |
| 4 | ICMP | Server-PT → PC1 | ICMP Echo Reply: Src = 1.1.1.1, confirming server received the ping |

🔧 Try It Yourself — Inspect DNS Packet Details In Simulation Mode, click on a DNS packet and open its PDU details. Navigate to the DNS layer and expand the 'Query' and 'Answer' sections. You should see: Query Name = youtube.com, Query Type = A (1), Answer IP = 1.1.1.1 Screenshot this expanded view and include it in your submission.

📸 SCREENSHOT REQUIRED — Simulation: DNS Query PDU Take a screenshot of the DNS Query PDU Details (Outbound PDU Details). Must show: Source IP = 192.168.0.1, Destination IP = 1.1.1.1, Port = 53, Query name = youtube.com Label it: 'Screenshot 13 — Simulation Mode: DNS Query PDU'

📸 SCREENSHOT REQUIRED — Simulation: DNS Reply PDU Take a screenshot of the DNS Reply PDU Details showing the Answer section. Must show: youtube.com → 1.1.1.1 in the Answer record. Label it: 'Screenshot 14 — Simulation Mode: DNS Reply PDU'

📝 Task 4 Questions — Answered Q(a): In the DNS Query PDU, what are the Source and Destination IP addresses? Source IP: 192.168.0.1 (PC1) Destination IP: 1.1.1.1 (Server-PT / DNS Server) Port: 53 (UDP — DNS uses port 53) Q(b): In the DNS Reply PDU, what hostname-to-IP mapping appears in the Answer section? Hostname: youtube.com Resolved IP: 1.1.1.1 Q(c): After DNS resolved the name, what destination IP did the ICMP ping go to? ICMP Destination IP: 1.1.1.1 Q(d): Why does the DNS Query use UDP and not TCP? Explain in one sentence: Answer: DNS queries use UDP because they are small, single-request/response exchanges that do not require the overhead of TCP's three-way handshake, connection management, or guaranteed delivery — UDP's speed makes it ideal for the short, low-latency lookups DNS performs on port 53. Q(e): If the INTERNET router did NOT have the static route back to 192.168.0.0/24, what would happen to the DNS Reply packet? Why? Answer: The DNS Reply packet from Server-PT (source: 1.1.1.1, destination: 192.168.0.1) would arrive at the INTERNET router on G0/1. However, without a route to 192.168.0.0/24, the INTERNET router's routing table would have no matching entry for 192.168.0.x. The packet would be dropped and the INTERNET router would send an 'ICMP Destination Unreachable' message back to Server-PT. PC1 would never receive the DNS Reply, meaning DNS name resolution would fail — and the subsequent ICMP ping to youtube.com would also never be triggered. This is exactly why the static route 'ip route 192.168.0.0 255.255.255.0 203.0.113.1' is required on the INTERNET router.

# 6. Deliverables & Submission

Submit all of the following via LMS before the deadline. Each deliverable is listed with its exact screenshot label to use in your submission.

| ✓ | # | Deliverable | Details & Screenshot Label |
| --- | --- | --- | --- |
| ✅ | 1 | .pkt File | YourName_RollNo_DNSLab.pkt — fully working topology. |
| ✅ | 2 | Full Topology Screenshot | All 7 devices placed and labelled, all cables connected, all link lights green. |
| ✅ | 3 | Server-PT DNS Record Screenshot | Services → DNS tab showing youtube.com A record = 1.1.1.1. (Screenshot 2) |
| ✅ | 4 | Server-PT HTTP Page Screenshot | Services → HTTP tab showing custom index.html content. (Screenshot 3) |
| ✅ | 5 | INTERNET Router: show ip interface brief | Both interfaces up/up with correct IPs. (Screenshot 4) |
| ✅ | 6 | INTERNET Router: show ip route | Showing 2 connected routes + static to 192.168.0.0/24. (Screenshot 5) |
| ✅ | 7 | R1: show ip route | Showing S* default route + 2 connected routes. (Screenshot 6) |
| ✅ | 8 | R1: show hosts | Showing all 4 permanent host entries (R1, PC1, PC2, PC3). (Screenshot 7) |
| ✅ | 9 | R1: ping PC1 by name | Screenshot of 'ping PC1' from R1 CLI — must show 5 replies. (Screenshot 12) |
| ✅ | 10 | PC1: all ping tests | Screenshot showing pings to 192.168.0.254, 1.1.1.1, and youtube.com. (Screenshot 10) |
| ✅ | 11 | PC1: Web Browser | Screenshot of http://youtube.com loaded in PC1's Desktop Browser. (Screenshot 11) |
| ✅ | 12 | Simulation Mode: DNS Query PDU | Screenshot of PDU details showing Src/Dst IP, port 53, query name. (Screenshot 13) |
| ✅ | 13 | Simulation Mode: DNS Reply PDU | Screenshot of PDU Answer section showing youtube.com → 1.1.1.1. (Screenshot 14) |
| ✅ | 14 | Completed Task Questions | All 4 task question boxes filled in — completed in this document. |

⚠️ Submission Warning Submissions after the deadline will be penalised 50% per day. Identical .pkt files between students will be treated as plagiarism — automatic F grade per course policy. Ensure your .pkt file is renamed: YourName_RollNo_DNSLab.pkt before uploading.

# 7. Grading Rubric

Total: 20 Marks  |  CLO 4 — Configure network services in Cisco Packet Tracer  [Bloom's Apply — C3]

| Criterion | Marks | Full Marks | Partial | Needs Work |
| --- | --- | --- | --- | --- |
| Server-PT Configuration (DNS + Web Server) | 4 | Correct IP, DNS ON with youtube.com A record → 1.1.1.1, HTTP ON with custom page. Both screenshots provided. | One service correctly configured or A record points to wrong IP. | Server not configured or DNS/HTTP both off. |
| INTERNET Router Configuration | 3 | Both interfaces up/up. Correct IPs. Static route to 192.168.0.0/24 present. Ping to 1.1.1.1 succeeds. | Interfaces correct but static route missing or one interface has wrong IP. | Router not configured or both interfaces down. |
| R1 Configuration (Interfaces, Route, DNS, Hosts) | 5 | G0/0 and G0/1 up/up. S* default route present. ip name-server 1.1.1.1 set. All 4 ip host entries in 'show hosts'. | Interfaces and route correct but 1-2 ip host entries missing or ip name-server not set. | R1 interfaces missing IPs or default route absent. |
| Task 3 — Ping by Hostname from R1 | 3 | R1 pings PC1, PC2, PC3 by hostname — all succeed. R1 pings youtube.com successfully via DNS. | 1-2 hostname pings succeed. youtube.com ping fails or missing screenshot. | All hostname pings fail or no CLI screenshots. |
| Task 4 — Simulation Mode Analysis | 3 | PC1 ping youtube.com shows [1.1.1.1]. Simulation screenshots show DNS Query and DNS Reply PDUs. Questions answered. | Ping works but simulation screenshots missing key PDU details. Questions partially answered. | Ping by name fails or simulation mode not used. |
| Task Questions & Understanding | 2 | All 4 task question boxes fully and correctly answered. Demonstrates clear understanding. | 2-3 question boxes answered correctly. Some answers vague. | Fewer than 2 questions answered or mostly incorrect. |

Query it, resolve it, browse it — that is how engineers learn.

Instructor: Shazia Rizwan   |   Lab Instructor: Hasnain Ahmad   |   Spring 2026, BNU SCIT