Saad Mughal

F2023-009

Total Marks: 10

Due 29 April 2026

# Computer Networks LAB

# Week 10

# Network Configuration

Course Lecturer: Shaiza Rizwan

Lab Instructor: Hasnain Ahmad

Section: C

# Computer Networks LAB (LAB-209) - Lab 08

Network Configuration

## Task 1-5

## Task 6

- Write down the exact commands you used to set the hostname and passwords on HQ-Router:

Router> enable

Router# configure terminal

Router(config)# hostname HQ-Router

HQ-Router(config)# enable secret class

HQ-Router(config)# line console 0

HQ-Router(config-line)# password cisco

HQ-Router(config-line)# login

HQ-Router(config-line)# exit

- Write down the exact commands you used to assign the IP address to GigabitEthernet0/0:

HQ-Router(config)# interface gig

HQ-Router(config)# interface GigabitEthernet0/0

HQ-Router(config-if)# ip address 10.0.0.1 255.255.255.0

HQ-Router(config-if)# no shutdown

HQ-Router(config-if)# exit

Write memory

- Copy the output of 'show ip interface brief' from your router CLI here:

- Ping Test Results:

| Ping Test | Packet Sent | Packet Received | Pass/Fail |
| --- | --- | --- | --- |
| PC-Admin pings Router (10.0.0.1) | 4 | 4 | Pass |
| PC-Sales pings Router (10.0.0.1) | 4 | 4 | Pass |
| PC-Admin pings PC-Sales (10.0.0.20) | 4 | 4 | Pass |
| PC-Sales pings PC-Admin (10.0.0.10) | 4 | 4 | Pass |

- From 'show running-config', write the lines that confirm your hostname and passwords

HQ-Router#show running-config

Building configuration...

Current configuration : 784 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname HQ-Router

!

!

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

!

!

!

!

ip cef

no ipv6 cef