BEACONHOUSE NATIONAL UNIVERSITY

School of Computing & Information Technology

LAB ASSIGNMENT 2

DHCP Server, Client & Relay Agent

Using Cisco Packet Tracer

| Course | Computer Networks Lab (Lab-209) |
| --- | --- |
| Semester | Spring 2026 |
| Institution | Beaconhouse National University |
| Instructor | Shazia Rizwan |
| Lab Instructor | Hasnain Ahmad |
| Total Marks | 20 Marks |
| CLOs Covered | CLO 4 (Apply — C3) |
| Submission | Submit .pkt file + screenshots via LMS |

Student Name: ___________________________     Roll No: ________________     Section: ______

# 1. Learning Objectives

By the end of this lab, you will be able to:

- Explain the role of DHCP in automating IP address assignment across a network

- Configure a DHCP server on a Cisco router with multiple address pools

- Exclude specific IP addresses from DHCP pools to reserve them for static devices

- Configure a router interface as a DHCP client to receive an IP address automatically

- Set up a DHCP Relay Agent (ip helper-address) to forward DHCP requests across subnets

- Verify DHCP operation from both the server side (show run) and client side (ipconfig)

Mapped CLO:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — Bloom's C3]

# 2. Background — DHCP Concepts

Dynamic Host Configuration Protocol (DHCP) is a network service that automatically assigns IP addresses, subnet masks, default gateways, and DNS server addresses to hosts. Instead of manually configuring every device, a DHCP server handles addressing dynamically.

| Concept | Explanation |
| --- | --- |
| DHCP Server | A router or device configured to hand out IP addresses from defined pools to requesting clients. |
| DHCP Pool | A named range of IP addresses on a specific subnet that the server assigns from. Each subnet needs its own pool. |
| Excluded Addresses | IPs reserved from a pool so the server never assigns them — used for routers, servers, and printers that need static IPs. |
| DHCP Client | A device (PC or router interface) that sends a DHCP Discover broadcast to automatically obtain an IP. |
| DHCP Relay Agent | When a DHCP server is on a different subnet than the client, the router uses 'ip helper-address' to forward DHCP broadcasts across subnets. |
| ip helper-address | The Cisco IOS command placed on the router interface closest to clients, pointing at the DHCP server's IP. It converts the broadcast to a unicast. |

# 3. Lab Topology

The topology below shows the complete network you will build in Cisco Packet Tracer. Study it carefully before beginning — understanding the layout will make every configuration step easier.

Figure 1 — DHCP Lab Topology

## 3.1  Network Summary

| Device / Link | Interface | Network / IP | Role |
| --- | --- | --- | --- |
| R2 ←→ SW2 / PC2 | G0/1 | 192.168.2.0 /24 | DHCP Server — Pool2 for PC2 |
| R2 ←→ R1 | G0/0 | 203.0.113.0 /30 | WAN Link — Pool3; R1 G0/0 gets IP via DHCP |
| R1 ←→ SW1 / PC1 | G0/1 | 192.168.1.0 /24 | Relay Agent subnet — Pool1 for PC1 |
| PC2 | NIC | DHCP from Pool2 | DHCP client — served directly by R2 |
| R1 G0/0 | G0/0 | DHCP from Pool3 | Router interface as DHCP client |
| PC1 | NIC | DHCP from Pool1 | DHCP client — served via Relay Agent on R1 |

💡 Tip R2 is the DHCP Server for the entire network. R1 is both a DHCP client (on its G0/0 toward R2) and a DHCP Relay Agent (on its G0/1 toward PC1).

# 4. Lab Tasks

This lab has 4 tasks. The CLO mapping is shown next to each task heading. Complete them in order.

Task 1 — Configure DHCP Pools on R2

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

R2 is your DHCP server. You will create three pools and exclude reserved addresses before any IPs are assigned.

### Task 1.1 — Exclude Reserved IP Addresses

Before creating pools, exclude the IPs that must stay static (router interfaces, etc.). Exclusions must be set first, before the pools are defined.

! Enter privileged and configuration mode on R2 R2> enable R2# configure terminal ! Exclude .1 to .10 from Pool1 (reserved for router/static devices in 192.168.1.0/24) R2(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.10 ! Exclude .1 to .10 from Pool2 (reserved for router/static devices in 192.168.2.0/24) R2(config)# ip dhcp excluded-address 192.168.2.1 192.168.2.10 ! Exclude R2's own WAN address from Pool3 (203.0.113.1) R2(config)# ip dhcp excluded-address 203.0.113.1

💡 Tip Always exclude addresses before defining pools. If a pool is created first, the server may hand out a reserved IP before the exclusion is applied.

### Task 1.2 — Create DHCP Pools

Now create three pools — one per subnet. Each pool needs a network statement and a default-router.

! ── Pool 1: for PC1 (192.168.1.0/24) ────────────────────────── R2(config)# ip dhcp pool Pool1 R2(dhcp-config)# network 192.168.1.0 255.255.255.0 R2(dhcp-config)# default-router 192.168.1.1 R2(dhcp-config)# dns-server 8.8.8.8 R2(dhcp-config)# domain-name jeremysitlab.com R2(dhcp-config)# exit ! ── Pool 2: for PC2 (192.168.2.0/24) ────────────────────────── R2(config)# ip dhcp pool Pool2 R2(dhcp-config)# network 192.168.2.0 255.255.255.0 R2(dhcp-config)# default-router 192.168.2.1 R2(dhcp-config)# dns-server 8.8.8.8 R2(dhcp-config)# domain-name jeremysitlab.com R2(dhcp-config)# exit ! ── Pool 3: for WAN link (203.0.113.0/30) R1's G0/0 gets IP from here R2(config)# ip dhcp pool Pool3 R2(dhcp-config)# network 203.0.113.0 255.255.255.252 R2(dhcp-config)# exit

⚠️ Note Pool3 uses a /30 subnet (255.255.255.252) — only 4 IPs total, 2 usable. The .1 is excluded (R2's address), so the only assignable IP is 203.0.113.2, which will go to R1's G0/0.

### Task 1.3 — Verify DHCP Configuration

Confirm your pools and exclusions are correctly saved on R2:

! Verify all DHCP configuration in the running config R2# do show run ¦ section dhcp

✅ Expected Result You should see all three pool definitions and the three excluded-address lines in the output. If a pool is missing, re-enter its configuration.

🔧 Try It Yourself Also run 'show ip dhcp pool' and 'show ip dhcp binding' on R2. What information do these commands show that 'show run' does not?

📝 Task 1 Question After running show ip dhcp binding, what IP address did PC2 receive? Write it here: IP received by PC2: _________________________

Task 2 — Configure R1's G0/0 as a DHCP Client

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

Instead of manually assigning an IP to R1's WAN interface (G0/0), you will configure it to obtain an IP automatically from R2's Pool3. This demonstrates that routers can act as DHCP clients too.

! Configure R1's G0/0 interface to receive its IP from DHCP (Pool3 on R2) R1> enable R1# configure terminal R1(config)# interface GigabitEthernet0/0 R1(config-if)# ip address dhcp R1(config-if)# no shutdown R1(config-if)# end ! Verify R1 received an IP on G0/0 R1# show ip interface brief

✅ Expected Result R1's GigabitEthernet0/0 should show an IP address of 203.0.113.2 (the only usable IP after 203.0.113.1 is excluded). Status should be 'up/up'.

⚠️ Note The 'no shutdown' command is essential — interfaces are administratively down by default on Cisco routers and must be explicitly enabled.

📝 Task 2 Question What IP address did R1's G0/0 interface receive from R2's DHCP Pool3? Why is this the only possible IP it could receive? Answer: _________________________________________________________________

Task 3 — Configure R1 as a DHCP Relay Agent

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

PC1 is on the 192.168.1.0/24 subnet connected to R1. The DHCP server (R2) is on a different subnet. By default, routers drop DHCP broadcast messages — so PC1 cannot reach R2 directly.

The solution is to configure R1 as a DHCP Relay Agent on its G0/1 interface. This tells R1 to intercept DHCP broadcasts from PC1 and forward them as unicast packets to R2's IP address.

! Configure R1's G0/1 interface as a DHCP Relay Agent ! This interface faces PC1 and SW1 R1(config)# interface GigabitEthernet0/1 ! Replace [R2's IP] with R2's actual G0/0 interface IP (203.0.113.1) R1(config-if)# ip helper-address 203.0.113.1 R1(config-if)# end R1# write memory

💡 Tip The ip helper-address must point to the DHCP server's IP — in this case R2's WAN interface (203.0.113.1). If you point it to the wrong address, PC1's DHCP request will never reach R2.

🔧 Try It Yourself Before configuring ip helper-address, try requesting a DHCP address from PC1 (see Task 4). What happens? Does it succeed? Then apply ip helper-address and try again. Observe the difference.

📝 Task 3 Question In your own words, explain why 'ip helper-address' is needed in this topology. What problem does it solve? Answer: _________________________________________________________________ _________________________________________________________________________

Task 4 — Request IP Addresses from PC1 and PC2

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

Now trigger the DHCP process from both PCs and verify they receive correct IP addresses from their respective pools.

### From PC2 — Direct DHCP (Pool2)

PC2 is on the same subnet as R2, so it will get its IP directly without a relay agent.

! Open the Command Prompt on PC2 (Desktop → Command Prompt) ! Request a new IP address from the DHCP server C:\> ipconfig /renew ! Verify full details including DNS and domain C:\> ipconfig /all

✅ Expected Result PC2 should receive an IP in the range 192.168.2.11–254 (since .1–.10 are excluded), with Default Gateway 192.168.2.1, DNS 8.8.8.8, and Domain jeremysitlab.com.

### From PC1 — DHCP via Relay Agent (Pool1)

PC1 needs R1 to relay its DHCP request to R2. Ensure Task 3 is completed before running these commands.

! Open the Command Prompt on PC1 (Desktop → Command Prompt) ! View current IP settings first C:\> ipconfig /all ! Request a new IP address — this will be relayed by R1 to R2 C:\> ipconfig /renew ! Confirm the received IP and full details C:\> ipconfig /all

✅ Expected Result PC1 should receive an IP in the range 192.168.1.11–254, with Default Gateway 192.168.1.1, DNS 8.8.8.8, and Domain jeremysitlab.com. If PC1 gets 0.0.0.0, check ip helper-address on R1's G0/1.

🔧 Try It Yourself After both PCs have IPs, run 'show ip dhcp binding' on R2 again. You should now see entries for both PC1 and PC2. Can you identify which pool each entry came from based on the IP address?

📝 Task 4 Question Record the IP addresses received by each device below. Take a screenshot of ipconfig /all for both PCs and attach it to your submission. Device; IP Address Received; Default Gateway; Pool Used. PC1. PC2.

# 5. Deliverables & Submission

Submit all of the following via LMS before the deadline:

| # | Deliverable | Details |
| --- | --- | --- |
| 1 | Packet Tracer File (.pkt) | YourName_RollNo_DHCPLab.pkt — fully configured topology. |
| 2 | Topology Screenshot | Full workspace with all devices, cables, and green link lights. |
| 3 | 'show run ¦ section dhcp' Output | Screenshot from R2's CLI showing all three pools and exclusions. |
| 4 | 'show ip dhcp binding' Output | Screenshot from R2 after both PCs have leased IPs. |
| 5 | 'show ip interface brief' from R1 | Screenshot showing R1's G0/0 with assigned IP (up/up). |
| 6 | ipconfig /all from PC1 | Screenshot of Command Prompt showing IP, gateway, DNS, domain. |
| 7 | ipconfig /all from PC2 | Screenshot of Command Prompt showing IP, gateway, DNS, domain. |
| 8 | Completed Answer Questions | Answers to all four Task Questions filled in on this document. |

⚠️ Note Submissions received after the deadline will be penalised 50% per day. Identical .pkt files between students will be treated as plagiarism and result in an automatic F grade as per course policy.

# 6. Grading Rubric

Total: 20 Marks   |   CLO: CLO 4 — Configure network services in Cisco Packet Tracer   [Bloom's Apply — C3]

| Criterion | Marks | Full Marks All criteria met | Partial Minor errors | Needs Work Major errors / missing |
| --- | --- | --- | --- | --- |
| Task 1 — DHCP Pool Configuration on R2 | 6 | All 3 pools defined correctly with right network, gateway, DNS, domain. All 3 exclusions in place. 'show run' confirms. (6 marks) | 1–2 pools correct. Minor error in exclusion or missing DNS/domain in one pool. (3–5 marks) | Pools missing or incorrectly configured. Exclusions absent. (0–2 marks) |
| Task 2 — R1 G0/0 as DHCP Client | 4 | G0/0 configured with 'ip address dhcp' and 'no shutdown'. Interface shows up/up with 203.0.113.2 in 'show ip interface brief'. (4 marks) | Command entered but interface still down, or wrong IP received. (2–3 marks) | G0/0 not configured as DHCP client or interface is down. (0–1 mark) |
| Task 3 — DHCP Relay Agent on R1 | 4 | 'ip helper-address 203.0.113.1' correctly applied on G0/1. PC1 successfully receives IP via relay. (4 marks) | helper-address applied but pointing to wrong IP, or PC1 still fails to get IP. (2–3 marks) | ip helper-address missing or G0/1 not configured. (0–1 mark) |
| Task 4 — PC IP Verification | 4 | Both PC1 and PC2 show correct IPs, gateways, DNS, and domain from 'ipconfig /all'. Screenshots submitted. (4 marks) | One PC gets correct IP; other has partial or incorrect config. (2–3 marks) | Both PCs fail to receive IPs or no screenshots submitted. (0–1 mark) |
| Task Questions & Understanding | 2 | All 4 task questions answered correctly and in student's own words. Answers show understanding of DHCP concepts. (2 marks) | 2–3 questions answered correctly or answers are vague. (1 mark) | Questions not answered or mostly incorrect. (0 marks) |
| TOTAL | 20 Marks |  |  |  |

# 7. CLO Mapping Summary

| CLO | Description | Tasks Covered | Bloom's Level | Marks |
| --- | --- | --- | --- | --- |
| CLO 4 | Configure network services in Cisco Packet Tracer | Tasks 1–4: DHCP pools, exclusions, relay agent, client config & verification | Apply (C3) | 20 / 20 |

Configure it, verify it, understand it — that is how engineers learn. Instructor: Shazia Rizwan ¦ Lab Instructor: Hasnain Ahmad ¦ Spring 2026, BNU SCIT