BEACONHOUSE NATIONAL UNIVERSITY

School of Computing & Information Technology

LAB ASSIGNMENT 5

Routing Protocol Redistribution

RIP  ⟷  OSPF  |  Cisco Packet Tracer

| Course | Computer Networks Lab (Lab-209) |
| --- | --- |
| Semester | Spring 2026 |
| Institution | Beaconhouse National University |
| Instructor | Shazia Rizwan |
| Lab Instructor | Hasnain Ahmad |
| Total Marks | 20 Marks |
| CLOs Covered | CLO 2 (Analyze — C4) ¦ CLO 4 (Apply — C3) |
| Submission | Submit .pkt file + screenshots via LMS |

Student Name: ___________________________     Roll No: ________________     Section: ______

# 1. Learning Objectives

By the end of this lab, you will be able to:

- Explain why routing protocol redistribution is needed in multi-protocol networks

- Understand the difference between RIP (distance vector) and OSPF (link-state) routing protocols

- Configure RIP on the NayaTel domain routers

- Configure OSPF on the PTCL domain routers

- Configure Route Redistribution on the edge router (Router2) to share routes between both domains

- Verify end-to-end connectivity between PC0 (RIP domain) and PC2/Web Server (OSPF domain)

- Analyse routing tables to confirm redistributed routes appear correctly

Mapped CLOs:  CLO 2 — Compare TCP/UDP protocol implementations  [Analyze — C4]    |  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

# 2. Background

In real-world networks, different organisations and ISPs run different routing protocols. NayaTel may use RIP internally while PTCL uses OSPF. When these two networks need to exchange routes — for example so a customer of NayaTel can access a web server hosted on PTCL's network — the edge router between them must perform Route Redistribution.

Route Redistribution is the process of taking routes learned by one routing protocol and injecting them into another routing protocol's domain. Without it, the two domains are completely isolated and cannot communicate.

| Concept | Explanation |
| --- | --- |
| RIP | Routing Information Protocol. A distance-vector protocol that uses hop count as its metric. Simple to configure but limited to 15 hops maximum — it cannot handle networks larger than 15 routers. |
| OSPF | Open Shortest Path First. A link-state protocol that uses cost (based on bandwidth) as its metric. Scales to very large networks, converges faster than RIP, and has no 15-hop limitation. |
| Redistribution | The process of importing routes from one routing protocol into another. An edge router running both protocols acts as the bridge between the two domains. |
| Edge Router | Router2 in this lab. It runs both RIP and OSPF simultaneously and redistributes routes in both directions — injecting OSPF routes into RIP and RIP routes into OSPF. |
| Metric | A value assigned to redistributed routes so the receiving protocol knows their cost. RIP uses hop count (metric 10 in this lab), OSPF uses subnets keyword to include all subnet types. |
| NayaTel Domain | Switch0 → Router0 → Router1 → Router2. This segment runs RIP. Represents an ISP's internal network. |
| PTCL Domain | Router2 → Router3 → Switch1 → Web Server + PC2. This segment runs OSPF. Represents a different ISP's network. |

# 3. Lab Topology

Study the topology carefully. The red line separating Router2 shows the redistribution boundary — the point where two different routing protocols meet.

Figure 1 — RIP & OSPF Redistribution Topology

## 3.1  Domain Summary

| Domain | Protocol | Devices | Notes |
| --- | --- | --- | --- |
| NayaTel (left side) | RIP | Switch0, Router0, Router1, Router2 (G0/1 side) | Max 15 hops. Simple but limited. |
| Re-Distribution Point | RIP + OSPF | Router2 (edge router — runs both) | Redistributes in both directions. |
| PTCL (right side) | OSPF Area 0 | Router3, Switch1, Web Server, PC2 | Scalable, no hop limit. |

## 3.2  IP Address Plan

| Device | Interface | IP Address | Subnet Mask | Connected To |
| --- | --- | --- | --- | --- |
| PC0 | NIC | 192.168.1.x | 255.255.255.0 | Switch0 |
| Switch0 | — | — | — | PC0 + Router0 G0/0 |
| Router0 | G0/0 | 192.168.1.1 | 255.255.255.0 | Switch0 (LAN A) |
| Router0 | G0/1 | 10.0.1.1 | 255.255.255.0 | Router1 G0/0 |
| Router1 | G0/0 | 10.0.1.2 | 255.255.255.0 | Router0 G0/1 |
| Router1 | G0/1 | 10.0.2.1 | 255.255.255.0 | Router2 G0/0 |
| Router2 | G0/0 | 10.0.2.2 | 255.255.255.0 | Router1 G0/1 (RIP side) |
| Router2 | G0/1 | 10.0.3.1 | 255.255.255.0 | Router3 G0/0 (OSPF side) |
| Router3 | G0/0 | 10.0.3.2 | 255.255.255.0 | Router2 G0/1 |
| Router3 | G0/1 | 192.168.3.1 | 255.255.255.0 | Switch1 (LAN B) |
| Switch1 | — | — | — | Router3 + Web Server + PC2 |
| Web Server | NIC | 192.168.3.x | 255.255.255.0 | Switch1 |
| PC2 | NIC | 192.168.3.x | 255.255.255.0 | Switch1 |

⚠️ Note Router2 is the ONLY router that runs both RIP and OSPF. All other routers run only one protocol. This is a critical concept — redistribution only happens at the edge router.

# 4. Lab Tasks — Follow Along

This is a follow-along lab. Complete every task in order. Each task builds directly on the previous one — skipping ahead will cause the lab to fail.

Task 1 — Set Up the Topology and Assign IP Addresses

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

Before any routing protocol can work, every router interface must have the correct IP address assigned and be activated. Open Cisco Packet Tracer and build the topology from scratch, or open the provided .pkt file and verify all IPs.

### Step 1.1 — Configure Router0

! Configure Router0 — NayaTel LAN gateway + first RIP hop Router> enable Router# configure terminal Router(config)# hostname Router0 ! G0/0 faces Switch0 and PC0 (LAN A) Router0(config)# interface GigabitEthernet0/0 Router0(config-if)# ip address 192.168.1.1 255.255.255.0 Router0(config-if)# no shutdown Router0(config-if)# exit ! G0/1 faces Router1 (WAN link 10.0.1.0/24) Router0(config)# interface GigabitEthernet0/1 Router0(config-if)# ip address 10.0.1.1 255.255.255.0 Router0(config-if)# no shutdown Router0(config-if)# exit Router0(config)# end Router0# write memory

### Step 1.2 — Configure Router1

! Router1 — middle hop in the RIP domain Router> enable Router# configure terminal Router(config)# hostname Router1 ! G0/0 faces Router0 Router1(config)# interface GigabitEthernet0/0 Router1(config-if)# ip address 10.0.1.2 255.255.255.0 Router1(config-if)# no shutdown Router1(config-if)# exit ! G0/1 faces Router2 (WAN link 10.0.2.0/24) Router1(config)# interface GigabitEthernet0/1 Router1(config-if)# ip address 10.0.2.1 255.255.255.0 Router1(config-if)# no shutdown Router1(config-if)# exit Router1(config)# end Router1# write memory

### Step 1.3 — Configure Router2 (Edge Router)

! Router2 — redistribution point. Has one RIP-side and one OSPF-side interface Router> enable Router# configure terminal Router(config)# hostname Router2 ! G0/0 faces Router1 — RIP domain side Router2(config)# interface GigabitEthernet0/0 Router2(config-if)# ip address 10.0.2.2 255.255.255.0 Router2(config-if)# no shutdown Router2(config-if)# exit ! G0/1 faces Router3 — OSPF domain side Router2(config)# interface GigabitEthernet0/1 Router2(config-if)# ip address 10.0.3.1 255.255.255.0 Router2(config-if)# no shutdown Router2(config-if)# exit Router2(config)# end Router2# write memory

### Step 1.4 — Configure Router3

! Router3 — PTCL gateway router, OSPF domain Router> enable Router# configure terminal Router(config)# hostname Router3 ! G0/0 faces Router2 Router3(config)# interface GigabitEthernet0/0 Router3(config-if)# ip address 10.0.3.2 255.255.255.0 Router3(config-if)# no shutdown Router3(config-if)# exit ! G0/1 faces Switch1, Web Server and PC2 (LAN B) Router3(config)# interface GigabitEthernet0/1 Router3(config-if)# ip address 192.168.3.1 255.255.255.0 Router3(config-if)# no shutdown Router3(config-if)# exit Router3(config)# end Router3# write memory

### Step 1.5 — Configure PC0, PC2, and Web Server

Set static IPs on all end devices. Click each device → Desktop → IP Configuration → Static.

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |
| Web Server | 192.168.3.20 | 255.255.255.0 | 192.168.3.1 |

✅ Expected Result After completing Task 1, run 'show ip interface brief' on each router. All interfaces should show 'up/up'. Ping between directly connected routers (e.g. Router0# ping 10.0.1.2) to confirm basic connectivity before adding routing protocols.

📝 Task 1 Question What is the IP address of Router2's G0/1 interface? _______________ What two domains does Router2 connect? ________________ and ________________

Task 2 — Configure RIP on the NayaTel Domain

CLO Mapped:  CLO 2 — Compare TCP/UDP protocol implementations  [Analyze — C4]    |  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

📖 Key Concept RIP (Routing Information Protocol) must be configured on Router0, Router1, and the RIP-side of Router2. For each router, you tell RIP which directly connected networks to advertise using the 'network' command. RIP then automatically shares these networks with its neighbours.

### Step 2.1 — Configure RIP on Router0

! Enable RIP and advertise Router0's directly connected networks Router0# configure terminal Router0(config)# router rip Router0(config-router)# version 2 Router0(config-router)# network 192.168.1.0 Router0(config-router)# network 10.0.1.0 Router0(config-router)# no auto-summary Router0(config-router)# exit Router0(config)# end Router0# write memory

### Step 2.2 — Configure RIP on Router1

! Router1 sits between Router0 and Router2 — advertise both its networks Router1# configure terminal Router1(config)# router rip Router1(config-router)# version 2 Router1(config-router)# network 10.0.1.0 Router1(config-router)# network 10.0.2.0 Router1(config-router)# no auto-summary Router1(config-router)# exit Router1(config)# end Router1# write memory

### Step 2.3 — Configure RIP on Router2 (RIP side only)

! Router2 only advertises its RIP-side interface (G0/0) in RIP ! Do NOT add 10.0.3.0 here — that is the OSPF side Router2# configure terminal Router2(config)# router rip Router2(config-router)# version 2 Router2(config-router)# network 10.0.2.0 Router2(config-router)# no auto-summary Router2(config-router)# exit Router2(config)# end Router2# write memory

⚠️ Note Do NOT add the OSPF-side network (10.0.3.0) to RIP on Router2. That network belongs to OSPF. Adding it to RIP would bypass redistribution and create routing loops.

### Step 2.4 — Verify RIP

! On Router0 — check that it learned RIP routes from Router1 and Router2 Router0# show ip route rip ! Expected — should see R entries for 10.0.2.0/24 via Router1 R 10.0.2.0/24 [120/1] via 10.0.1.2, ... ! Also verify routing table on Router1 Router1# show ip route ! Test connectivity within RIP domain — ping Router2 from Router0 Router0# ping 10.0.2.2

✅ Expected Result Router0 should see 10.0.2.0/24 as an 'R' (RIP) route. The ping from Router0 to 10.0.2.2 (Router2's G0/0) should succeed with 5 replies. If it fails, verify RIP is configured on all three routers and 'no auto-summary' is set.

🔧 Try It Yourself Run 'show ip route' on Router1. How many RIP routes (marked R) does it see? Can you identify which networks it learned from Router0 and which from Router2?

📝 Task 2 Question What letter appears next to RIP-learned routes in the routing table? ___ What is the administrative distance of RIP shown in the routing table (e.g. [120/x])? ___ Why should 10.0.3.0 NOT be added to RIP on Router2? Answer: _________________________________________________________________

Task 3 — Configure OSPF on the PTCL Domain

CLO Mapped:  CLO 2 — Compare TCP/UDP protocol implementations  [Analyze — C4]    |  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

📖 Key Concept OSPF uses a 'network' command with a wildcard mask and an area number. All routers in this lab are in Area 0 (the backbone area). Unlike RIP, OSPF uses cost (bandwidth-based) as its metric and converges much faster.

### Step 3.1 — Configure OSPF on Router2 (OSPF side only)

! Router2 only advertises its OSPF-side interface (G0/1) in OSPF ! Process ID 1 — must match on all OSPF routers in the same domain Router2# configure terminal Router2(config)# router ospf 1 Router2(config-router)# network 10.0.3.0 0.0.0.255 area 0 Router2(config-router)# exit Router2(config)# end Router2# write memory

💡 Tip The wildcard mask 0.0.0.255 is the inverse of 255.255.255.0. In OSPF, the wildcard tells the router which hosts in that network to include. 0.0.0.255 means: match any host in the 10.0.3.0 network.

### Step 3.2 — Configure OSPF on Router3

! Router3 advertises BOTH its interfaces in OSPF Router3# configure terminal Router3(config)# router ospf 1 Router3(config-router)# network 10.0.3.0 0.0.0.255 area 0 Router3(config-router)# network 192.168.3.0 0.0.0.255 area 0 Router3(config-router)# exit Router3(config)# end Router3# write memory

### Step 3.3 — Verify OSPF

! Check OSPF neighbours between Router2 and Router3 Router2# show ip ospf neighbor ! Expected — Router3 appears as FULL state neighbour: Neighbor ID Pri State Dead Time Address Interface 192.168.3.1 1 FULL/DR 00:00:39 10.0.3.2 Gi0/1 ! Check OSPF routes on Router2 Router2# show ip route ospf ! Expected — O entry for 192.168.3.0/24 via Router3 O 192.168.3.0/24 [110/2] via 10.0.3.2, ... ! Ping Router3 from Router2 to test OSPF connectivity Router2# ping 10.0.3.2 Router2# ping 192.168.3.1

✅ Expected Result Router2 and Router3 should form an OSPF FULL adjacency. Router2's routing table should show 192.168.3.0/24 as an 'O' (OSPF) route. Both pings should succeed.

📝 Task 3 Question What letter appears next to OSPF-learned routes in the routing table? ___ What is the administrative distance of OSPF shown in the routing table (e.g. [110/x])? ___ What OSPF neighbour state indicates a fully working adjacency? _______________ Compare OSPF and RIP: which has the lower administrative distance, and why does that matter? Answer: _________________________________________________________________

Task 4 — Configure Route Redistribution on Router2

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

This is the most important task in the lab. Router2 currently knows RIP routes on one side and OSPF routes on the other — but the two domains cannot communicate yet. Redistribution will bridge this gap by injecting each domain's routes into the other.

📖 Key Concept Redistribution is always configured on the edge router (Router2). It requires two commands: 1. Inside OSPF: import RIP routes → 'redistribute rip subnets' 2. Inside RIP: import OSPF routes → 'redistribute ospf 1 metric 10' The 'subnets' keyword in OSPF allows classless subnets to be redistributed. The 'metric 10' in RIP assigns a hop count to the imported OSPF routes.

### Step 4.1 — Redistribute RIP into OSPF (on Router2)

! This takes all RIP-learned routes and injects them into OSPF ! 'subnets' keyword is REQUIRED — without it, only classful networks are redistributed Router2# configure terminal Router2(config)# router ospf 1 Router2(config-router)# redistribute rip subnets Router2(config-router)# exit

### Step 4.2 — Redistribute OSPF into RIP (on Router2)

! This takes all OSPF-learned routes and injects them into RIP ! 'metric 10' assigns a RIP hop count of 10 to the redistributed OSPF routes Router2(config)# router rip Router2(config-router)# redistribute ospf 1 metric 10 Router2(config-router)# exit Router2(config)# end Router2# write memory

⚠️ Note Both redistribution directions are required. If you only do one direction, traffic can only flow one way — requests will go out but replies will never come back.

### Step 4.3 — Verify Redistribution

! On Router0 — check if OSPF routes now appear as RIP redistributed (R EX) Router0# show ip route ! Expected — should now see 192.168.3.0 and 10.0.3.0 as RIP entries: R 10.0.3.0/24 [120/10] via 10.0.1.2, ... R 192.168.3.0/24 [120/11] via 10.0.1.2, ... ! On Router3 — check if RIP routes now appear as OSPF External (O E2) Router3# show ip route ! Expected — should now see 192.168.1.0 and 10.0.1.0 as OSPF External: O E2 192.168.1.0/24 [110/20] via 10.0.3.1, ... O E2 10.0.1.0/24 [110/20] via 10.0.3.1, ... O E2 10.0.2.0/24 [110/20] via 10.0.3.1, ...

✅ Expected Result Router0 should now see 192.168.3.0/24 in its routing table (as a RIP route). Router3 should see 192.168.1.0/24 (as an OSPF External route marked O E2). This confirms redistribution is working in both directions.

📝 Task 4 Question After redistribution, what route marker appears on Router3 for routes learned from the RIP domain? Answer: _______________ What does the 'subnets' keyword do in the 'redistribute rip subnets' command? Answer: _________________________________________________________________ Why is 'metric 10' needed when redistributing OSPF routes into RIP? Answer: _________________________________________________________________

Task 5 — Verify End-to-End Connectivity

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]

With redistribution configured, PC0 in the NayaTel/RIP domain should now be able to reach PC2 and the Web Server in the PTCL/OSPF domain — and vice versa. Run the following tests to confirm full end-to-end connectivity.

### Step 5.1 — Ping Tests from PC0

! On PC0 → Desktop → Command Prompt ! Test 1: Ping Router0 gateway — confirms PC0's basic LAN connectivity C:\> ping 192.168.1.1 ! Test 2: Ping Router2 (edge router) by IP — crosses 2 RIP hops C:\> ping 10.0.2.2 ! Test 3: Ping Router3 — crosses redistribution boundary into OSPF domain C:\> ping 10.0.3.2 ! Test 4: Ping PC2 — full end-to-end across both domains C:\> ping 192.168.3.10 ! Test 5: Ping Web Server — confirms web server is reachable from RIP domain C:\> ping 192.168.3.20

### Step 5.2 — Ping Tests from PC2

! On PC2 → Desktop → Command Prompt ! Ping PC0 — cross-domain reverse direction test C:\> ping 192.168.1.10

### Step 5.3 — Traceroute from PC0 to PC2

! Traceroute shows every hop the packet takes ! Expected path: PC0 → Router0 → Router1 → Router2 → Router3 → PC2 C:\> tracert 192.168.3.10 ! Expected output: 1 <1 ms 192.168.1.1 (Router0 G0/0) 2 <1 ms 10.0.1.2 (Router1 G0/0) 3 <1 ms 10.0.2.2 (Router2 G0/0) 4 <1 ms 10.0.3.2 (Router3 G0/0) 5 <1 ms 192.168.3.10 (PC2)

✅ Expected Result All 5 ping tests from PC0 should succeed. The traceroute should show exactly 5 hops. This confirms that route redistribution is correctly sharing routes between the RIP and OSPF domains in both directions.

🔧 Try It Yourself Open the Web Server's HTTP service (Services → HTTP → ON). Then open PC0's Desktop → Web Browser and browse to 192.168.3.20. The web page should load — confirming full application-layer connectivity across both routing domains.

📝 Task 5 Question Did all 5 ping tests from PC0 succeed? Yes / No How many hops did the traceroute from PC0 to PC2 show? ___ Which router in the traceroute output represents the redistribution boundary? Answer: _______________ (IP: _______________ ) Did PC2 successfully ping PC0 in reverse? Yes / No

🏆 CHALLENGE TASK Add a New Router to the OSPF Domain and Verify Full Connectivity

CLO Mapped:  CLO 4 — Configure network services in Cisco Packet Tracer  [Apply — C3]    |  Marks: 5 (Bonus)

Now that you understand redistribution, apply your knowledge independently. Add a new router (Router4) to the OSPF domain and connect a new PC (PC3) to it. Then verify that PC0 (RIP domain) can ping PC3 (new OSPF device) without any changes to Router2's redistribution configuration.

🏆 Challenge Task What to do: 1. Add a new 2911 Router (Router4) to the workspace. Connect Router4's G0/0 to Router3's G0/2 using a Straight-Through cable. 2. Assign the following IPs: Router3 G0/2: 10.0.4.1 / 255.255.255.0 Router4 G0/0: 10.0.4.2 / 255.255.255.0 Router4 G0/1: 192.168.4.1 / 255.255.255.0 PC3 (new): 192.168.4.10 / 255.255.255.0 GW: 192.168.4.1 3. Add Router4's networks to OSPF Area 0 on Router4 and add the new link to OSPF on Router3. 4. Do NOT touch Router2's redistribution config — it should work automatically. 5. Verify: ping 192.168.4.10 from PC0. Does it succeed? Hint: OSPF will automatically share the new 192.168.4.0/24 route to Router3 → Router2 → and Router2 will redistribute it into RIP → Router1 → Router0 → PC0.

📝 Task Challenge Question What OSPF commands did you add to Router4? Command 1: _________________________________________________________________ Command 2: _________________________________________________________________ After adding Router4, did PC0 successfully ping PC3 (192.168.4.10)? Yes / No Did you need to change anything on Router2? Yes / No Why or why not? Answer: _________________________________________________________________ _________________________________________________________________________

# 5. Deliverables & Submission

Submit all of the following via LMS before the deadline:

| # | Deliverable | Details |
| --- | --- | --- |
| 1 | .pkt File | YourName_RollNo_Redistribution.pkt — fully working. |
| 2 | Full Topology Screenshot | All devices, cables, and green link lights visible. |
| 3 | Router0: show ip route | Showing RIP routes + redistributed OSPF routes (192.168.3.0). |
| 4 | Router3: show ip route | Showing OSPF routes + redistributed RIP routes (O E2 entries). |
| 5 | Router2: show ip ospf neighbor | Router3 showing FULL adjacency state. |
| 6 | PC0: All 5 ping tests | Screenshot of all 5 pings succeeding from PC0's Command Prompt. |
| 7 | PC0: tracert 192.168.3.10 | Screenshot showing all 5 hops of the traceroute. |
| 8 | PC0: Web Browser to 192.168.3.20 | Web page loaded successfully in PC0's Desktop browser. |
| 9 | Challenge Task (bonus) | Screenshot of PC0 pinging 192.168.4.10 + Router4 OSPF config. |
| 10 | Completed Task Questions | All 5 + Challenge question boxes answered on this document. |

⚠️ Note Submissions after the deadline: 50% penalty per day. Identical .pkt files = plagiarism = automatic F grade per course policy.

# 6. Grading Rubric

Total: 20 Marks  (+5 Bonus for Challenge Task)   |   CLOs: CLO 2 [C4]  |  CLO 4 [C3]

| Criterion | Marks | Full Marks All criteria met | Partial Minor errors | Needs Work Major errors / missing |
| --- | --- | --- | --- | --- |
| Tasks 1 — IP Configuration | 3 | All 4 routers and 3 end devices have correct IPs. All interfaces up/up. Direct pings between adjacent routers succeed. (3) | Most IPs correct but 1–2 interfaces have wrong IPs or are down. (1–2) | Multiple routers misconfigured or interfaces down. (0) |
| Task 2 — RIP Configuration | 4 | RIP v2 configured on Router0, Router1, Router2 (G0/0 side). 'no auto-summary' set. Router0 sees 10.0.2.0 as R route. (4) | RIP on 2 routers correct. auto-summary not disabled or one router missing. (2–3) | RIP not configured or no R routes visible. (0–1) |
| Task 3 — OSPF Configuration | 4 | OSPF process 1 on Router2 (G0/1) and Router3 (both interfaces). Router2 shows Router3 as FULL neighbour. 192.168.3.0 visible as O route on Router2. (4) | OSPF on one router correct. Neighbour formed but routes missing. (2–3) | OSPF not configured or adjacency not established. (0–1) |
| Task 4 — Redistribution | 5 | 'redistribute rip subnets' in OSPF on Router2. 'redistribute ospf 1 metric 10' in RIP on Router2. Router0 sees 192.168.3.0 as R. Router3 sees O E2 routes. (5) | One direction of redistribution correct. Other direction missing or misconfigured. (2–4) | Redistribution not configured. No cross-domain routes visible. (0–1) |
| Task 5 — End-to-End Verification | 4 | All 5 pings from PC0 succeed. Traceroute shows 5 correct hops. PC2 pings PC0. Web page loads in browser. Screenshots provided. (4) | 3–4 pings succeed. Traceroute partial or browser test missing. (2–3) | Cross-domain pings fail. No end-to-end connectivity. (0–1) |
| Challenge Task (Bonus) | 5 | Router4 added with correct IPs. OSPF configured on Router4. PC3 reachable from PC0 without any changes to Router2. Question box answered. (5) | Router4 added and configured but connectivity to PC0 fails. (2–4) | Challenge not attempted. (0) |
| TOTAL | 20 Marks + 5 Bonus Marks (Challenge Task) |  |  |  |

# 7. CLO Mapping Summary

| CLO | Description | Tasks Covered | Bloom's Level | Marks |
| --- | --- | --- | --- | --- |
| CLO 2 | Compare TCP/UDP protocol implementations | Tasks 2 & 3: RIP vs OSPF — compare metrics, AD, convergence, scalability | Analyze (C4) | 4 marks |
| CLO 4 | Configure network services in Cisco Packet Tracer | Tasks 1–5 + Challenge: IP config, RIP, OSPF, redistribution, verification | Apply (C3) | 16 marks + 5 bonus |

Two protocols, one network — redistribute it, verify it, extend it. Instructor: Shazia Rizwan ¦ Lab Instructor: Hasnain Ahmad ¦ Spring 2026, BNU SCIT