PROJECT PHASE I — CASE STUDY

BNU Campus Network — Existing Infrastructure Documentation

Student / Group Information

| Student Name(s) | Muhammad Arez Sheikh Muhammad Ibrahim Saad Mughal Muhammad Ismail Rana | Roll Number(s) | F2023-872 F2023-630 F2023-009 F2023-551 |
| --- | --- | --- | --- |
| Section | C | Group No. |  |
| Date | 17-5-26 | Instructor |  |

Marks Distribution

| # | Task | CLO | Marks | Obtained |
| --- | --- | --- | --- | --- |
| 0 | Group Formation & Role Assignment | — | 1 |  |
| 1 | Existing Network Study & IP Scheme Documentation | CLO-2 | 2 |  |
| 2 | Network Services Analysis (DHCP, DNS, NAT, VLAN) | CLO-1 | 1 |  |
| 3 | Problem Identification & Root Cause Analysis | CLO-2 | 2 |  |
| 4 | Constraints & Challenges Report (min. 10 issues) | CLO-3 | 2 |  |
| 5 | Proposed Solution & Improvement Recommendations | CLO-3 | 2 |  |
|  | TOTAL |  | 10 | / 10 |

Course Learning Outcomes (CLO) Mapping

| CLO | Description | Bloom's | Marks |
| --- | --- | --- | --- |
| CLO-1 | Implement client-server socket applications | Apply (C3) | 1 |
| CLO-2 | Compare TCP/UDP protocol implementations | Analyze (C4) | 4 |
| CLO-3 | Design concurrent servers using threads/forks | Create (C6) | 4 |

1.  University Network — Overview

Beaconhouse National University (BNU) operates a single-campus network connecting six administrative and academic departments. The network was set up incrementally over several years and is maintained by an in-house IT team. All departments are distributed across three main blocks and a dedicated Data Centre in the basement.

The network is built entirely on Cisco devices. A single core switch aggregates all departmental access switches and connects to one edge router for internet access. Inter-VLAN routing is handled by the router using Router-on-a-Stick. DHCP, DNS, LMS, and email services are hosted on servers in the Data Centre.

| Total Departments | 6 (Administrative & Academic) |
| --- | --- |
| Network Topology | Single-tier Star — all access switches connect to one core switch (CS-01) |
| Routing Method | Router-on-a-Stick — sub-interfaces on R-01 GigabitEthernet 0/1 |
| Core Switch | CS-01 — Cisco 3560-24PS |
| Edge Router | R-01 — Cisco 2911 |
| ISP Uplink | Single connection — Public IP 202.150.10.5 / 30 |
| Internal Address Space | 10.10.0.0 / 16 divided into /24 subnets, one per department |
| VLAN Range | VLAN 10 through VLAN 70 |
| DHCP | Centralised — Server (SRV-01, IP: 10.10.9.10) |
| DNS | Internal DNS on SRV-01, forwards to 8.8.8.8 for external names |
| NAT | PAT (overload) on R-01 Gi0/0 — all VLANs share one public IP |
| Wireless | One Access Point in Marketing department (VLAN 60, Block C, 1st Floor) |

2.  Core Infrastructure

2.1  Core Switch — CS-01

CS-01 is a Cisco 3560-24PS located in the Data Centre (Basement). It is the single aggregation point for all six departmental access switches and the server farm switch. All ports connected to other switches are configured as 802.1Q trunk ports.

| Hostname | CS-01 |
| --- | --- |
| Device | Cisco 3560-24PS |
| Location | Data Centre, Basement |
| Management IP | 10.10.0.1 / 255.255.255.0 |
| Uplink to Router | GigabitEthernet 0/1 → R-01 GigabitEthernet 0/1 (802.1Q trunk) |
| Downlinks to Depts | FastEthernet Fa0/1 – Fa0/7 (one port per switch) |
| VLANs Configured | VLAN 10, 20, 40, 50, 60, 70 |
| Inter-VLAN Routing | Disabled on CS-01 — all routing handled by R-01 |

2.2  Edge Router — R-01

R-01 is a Cisco 2911 in the Data Centre. GigabitEthernet 0/0 connects to the ISP. GigabitEthernet 0/1 trunks to CS-01 with one sub-interface per VLAN. PAT is applied on Gi0/0. DHCP relay (ip helper-address 10.10.9.10) is configured on every sub-interface so clients on all VLANs can reach the centralised DHCP server.

| Hostname | R-01 |
| --- | --- |
| Device | Cisco 2911 |
| WAN Interface | GigabitEthernet 0/0 — 202.150.10.5 / 30 (ISP assigned) |
| LAN Interface | GigabitEthernet 0/1 — 802.1Q trunk to CS-01 (no IP on physical interface) |
| Default Route | ip route 0.0.0.0 0.0.0.0 202.150.10.6 |
| NAT | ip nat inside source list 10 interface GigabitEthernet0/0 overload |
| NAT ACL | access-list 10 permit 10.10.0.0 0.0.255.255 |
| DHCP Relay | ip helper-address 10.10.9.10 on every sub-interface |
| Routing | Static routes only — no dynamic routing protocol |

R-01 Sub-interfaces  (Router-on-a-Stick)

| Sub-interface | VLAN | IP Address | Subnet Mask | Department |
| --- | --- | --- | --- | --- |
| Gi0/1.10 | 10 | 10.10.1.1 | 255.255.255.0 | Admission |
| Gi0/1.20 | 20 | 10.10.2.1 | 255.255.255.0 | Exam Department |
| Gi0/1.40 | 40 | 10.10.4.1 | 255.255.255.0 | Admin |
| Gi0/1.50 | 50 | 10.10.5.1 | 255.255.255.0 | Finance |
| Gi0/1.60 | 60 | 10.10.6.1 | 255.255.255.0 | Marketing |
| Gi0/1.70 | 70 | 10.10.7.1 | 255.255.255.0 | Server Farm |

2.3  Server Farm — Data Centre  (VLAN 70)

All servers reside in the Data Centre on VLAN 70. Each server has a manually assigned static IP address and is reachable from all other VLANs through R-01 sub-interface Gi0/1.70.

| Hostname | IP Address | Role / Notes |
| --- | --- | --- |
| SRV-01 | 10.10.7.10 (Static) | DHCP server for all VLANs + Primary DNS (internal zone: bnu.edu.pk) |
| SRV-02 | 10.10.7.11 (Static) | Web / LMS server — hosts the university Learning Management System |
| SRV-03 | 10.10.7.12 (Static) | Email server — handles all staff and student mail for bnu.edu.pk |
| SRV-04 | 10.10.7.13 (Static) | Backup server — scheduled backups from SRV-01, SRV-02, SRV-03 |

3.  VLAN Configuration Summary

VLANs are configured on CS-01. Each department has one dedicated VLAN. All inter-VLAN traffic is routed through R-01. No Access Control Lists (ACLs) are applied between any VLANs — all departments can reach each other freely through the router.

| VLAN ID | Name | Subnet | Gateway | DHCP Pool | ~Hosts |
| --- | --- | --- | --- | --- | --- |
| 10 | Admission | 10.10.1.0/24 | 10.10.1.1 | 10.10.1.100–200 | 30 |
| 20 | Exam | 10.10.2.0/24 | 10.10.2.1 | 10.10.2.100–200 | 25 |
| 40 | Admin | 10.10.4.0/24 | 10.10.4.1 | 10.10.4.100–200 | 20 |
| 50 | Finance | 10.10.5.0/24 | 10.10.5.1 | 10.10.5.100–180 | 20 |
| 60 | Marketing | 10.10.6.0/24 | 10.10.6.1 | 10.10.6.100–180 | 18 |
| 70 | Server Farm | 10.10.7.0/24 | 10.10.7.1 | Static only | — |

4.  DHCP Configuration

DHCP is hosted centrally on SRV-01 (10.10.7.10). Because SRV-01 is on VLAN 70 and clients are on VLANs 10–60, the command ip helper-address 10.10.7.10 is placed on every R-01 sub-interface to forward DHCP requests to SRV-01. All scopes share the same lease duration.

| Scope Name | Address Range | Subnet Mask | Gateway | DNS Server | Lease |
| --- | --- | --- | --- | --- | --- |
| VLAN10-Admission | 10.10.1.100–.200 | 255.255.255.0 | 10.10.1.1 | 10.10.7.10 | 8 Days |
| VLAN20-Exam | 10.10.2.100–.200 | 255.255.255.0 | 10.10.2.1 | 10.10.7.10 | 8 Days |
| VLAN40-Admin | 10.10.4.100–.200 | 255.255.255.0 | 10.10.4.1 | 10.10.7.10 | 8 Days |
| VLAN50-Finance | 10.10.5.100–.180 | 255.255.255.0 | 10.10.5.1 | 10.10.7.10 | 8 Days |
| VLAN60-Marketing | 10.10.6.100–.180 | 255.255.255.0 | 10.10.6.1 | 10.10.7.10 | 8 Days |

Devices listed as Static IP in each department section are manually configured and excluded from DHCP pools. This applies to all servers.

5.  DNS & NAT Configuration

5.1  DNS

| Primary DNS Server | SRV-01 — 10.10.7.10 |
| --- | --- |
| Secondary DNS Server | None configured |
| Internal Zone | bnu.edu.pk (hosted on SRV-01) |
| External Forwarder | 8.8.8.8 (Google Public DNS) |
| Sample Records | lms.bnu.edu.pk → 10.10.7.11 ¦ mail.bnu.edu.pk → 10.10.7.12 |
| Client Config | DNS address pushed to all clients via DHCP Option 6 |

5.2  NAT / PAT

R-01 performs PAT on interface GigabitEthernet 0/0. All internal traffic from 10.10.0.0/16 is translated to the single public IP 202.150.10.5 before going to the internet.

| NAT Type | PAT (overload) — all internal hosts share one public IP |
| --- | --- |
| Public IP | 202.150.10.5 / 30 |
| ISP Gateway | 202.150.10.6 |
| Inside Interface | R-01 Gi0/1 and all sub-interfaces (ip nat inside) |
| Outside Interface | R-01 Gi0/0 (ip nat outside) |
| NAT ACL | access-list 10 permit 10.10.0.0 0.0.255.255 |
| NAT Command | ip nat inside source list 10 interface GigabitEthernet0/0 overload |

6.  Departmental Network Details

The following sections describe the network configuration and devices for each of the six departments. All end-user computers are standard PCs or Laptops as available in Cisco Packet Tracer. All servers use the generic Server device in Cisco Packet Tracer.

6.1  Admission Department

| Block / Floor | Block A, Ground Floor |
| --- | --- |
| VLAN | VLAN 10 |
| Network | 10.10.1.0 / 24 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 10.10.1.1 (R-01 Gi0/1.10) |
| DHCP Pool | 10.10.1.100 – 10.10.1.200 |
| DNS Server | 10.10.7.10 |
| Access Switch | SW-ADM — Cisco 2960-24TT |
| Switch Uplink | SW-ADM Fa0/24 → CS-01 Fa0/1 (FastEthernet, 802.1Q trunk, VLAN 10) |
| Staff Count | ~30 users (admission officers + reception) |

Connected Devices:

| Hostname | IP Address | Role / Notes |
| --- | --- | --- |
| ADM-PC01 – ADM-PC08 | DHCP (.101–.108) | Admission officer workstations — student application processing |
| ADM-PC09 – ADM-PC12 | DHCP (.109–.112) | Reception terminals — student walk-in enquiries (public-facing) |
| ADM-LT01 | DHCP (.130) | Admission manager laptop |
| ADM-SRV01 | Static 10.10.1.10 | Local file server — admission forms and documents |

Note: Reception terminals ADM-PC09 to ADM-PC12 are used by visiting students. These machines are on the same VLAN as Admission staff workstations and the internal file server ADM-SRV01.

6.2  Exam Department

| Block / Floor | Block A, 1st Floor |
| --- | --- |
| VLAN | VLAN 20 |
| Network | 10.10.2.0 / 24 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 10.10.2.1 (R-01 Gi0/1.20) |
| DHCP Pool | 10.10.2.100 – 10.10.2.200 |
| DNS Server | 10.10.7.10 |
| Access Switch | SW-EXAM — Cisco 2960-24TT |
| Switch Uplink | SW-EXAM Fa0/24 → CS-01 Fa0/2 (FastEthernet, 802.1Q trunk, VLAN 20) |
| Staff Count | ~25 users (exam coordination staff) |

Connected Devices:

| Hostname | IP Address | Role / Notes |
| --- | --- | --- |
| EXAM-PC01 – EXAM-PC10 | DHCP (.101–.110) | Exam administration workstations |
| EXAM-PC11 – EXAM-PC15 | DHCP (.111–.115) | Result processing terminals |
| EXAM-SRV01 | DHCP (.150) | Exam database server — stores schedules, results, hall plans |
| EXAM-SRV02 | DHCP (.151) | Secondary exam server |

Note: EXAM-SRV01 and EXAM-SRV02 both receive IP addresses from the VLAN 20 DHCP pool. The exam scheduling software on staff workstations connects to EXAM-SRV01 using the hostname exam-db.bnu.edu.pk, resolved by SRV-01.

6.3  Admin Department

| Block / Floor | Block B, Ground Floor |
| --- | --- |
| VLAN | VLAN 40 |
| Network | 10.10.4.0 / 24 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 10.10.4.1 (R-01 Gi0/1.40) |
| DHCP Pool | 10.10.4.100 – 10.10.4.200 |
| DNS Server | 10.10.7.10 |
| Access Switch | SW-ADMIN — Cisco 2960-24TT |
| Switch Uplink | SW-ADMIN Fa0/24 → CS-01 Fa0/3 (FastEthernet, 802.1Q trunk, VLAN 40) |
| Staff Count | ~20 users (HR, facilities, general administration) |

Connected Devices:

| Hostname | IP Address | Role / Notes |
| --- | --- | --- |
| ADMIN-PC01 – ADMIN-PC10 | DHCP (.101–.110) | Admin and HR staff workstations |
| ADMIN-PC11 – ADMIN-PC15 | DHCP (.111–.115) | Facilities and support terminals |
| ADMIN-SRV01 | Static 10.10.4.10 | File server — HR documents and admin records |

Note: ADMIN-SRV01 is accessible as a shared drive from VLAN 40 workstations. Because no ACL restricts inter-VLAN access, staff from other VLANs can also reach this server through R-01.

6.4  Finance Department

| Block / Floor | Block B, 1st Floor |
| --- | --- |
| VLAN | VLAN 50 |
| Network | 10.10.5.0 / 24 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 10.10.5.1 (R-01 Gi0/1.50) |
| DHCP Pool | 10.10.5.100 – 10.10.5.180 |
| DNS Server | 10.10.7.10 |
| Access Switch | SW-FIN — Cisco 2960-24TT |
| Switch Uplink | SW-FIN Fa0/24 → CS-01 Fa0/4 (FastEthernet, 802.1Q trunk, VLAN 50) |
| Staff Count | ~20 users (payroll, budget, accounts) |

Connected Devices:

| Hostname | IP Address | Role / Notes |
| --- | --- | --- |
| FIN-PC01 – FIN-PC08 | DHCP (.101–.108) | Finance staff workstations — payroll and budget |
| FIN-PC09 – FIN-PC13 | DHCP (.109–.113) | Accounts processing terminals |

Note: Finance workstations run an ERP application that synchronises with SRV-02 (10.10.7.11) on port 8080 for leave data. All Finance PCs receive their IPs through DHCP.

6.5  Marketing Department

| Block / Floor | Block C, 1st Floor |
| --- | --- |
| VLAN | VLAN 60 |
| Network | 10.10.6.0 / 24 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 10.10.6.1 (R-01 Gi0/1.60) |
| DHCP Pool | 10.10.6.100 – 10.10.6.180 |
| DNS Server | 10.10.7.10 |
| Access Switch | SW-MKT — Cisco 2960-24TT |
| Switch Uplink | SW-MKT Fa0/24 → CS-01 Fa0/5 (FastEthernet, 802.1Q trunk, VLAN 60) |
| Wireless | One Access Point (AP-MKT) connected to SW-MKT Fa0/10 — SSID: BNU-Marketing |
| AP Encryption | WEP (configured on AP-MKT) |
| AP IP | Static 10.10.6.200 |
| Staff Count | ~18 users (wired + wireless) |

Connected Devices:

| Hostname | IP Address | Role / Notes |
| --- | --- | --- |
| MKT-PC01 – MKT-PC08 | DHCP (.101–.108) | Marketing staff — wired workstations |
| MKT-LT01 – MKT-LT04 | DHCP via WiFi (.110–.113) | Staff laptops connecting wirelessly via AP-MKT |
| AP-MKT | Static 10.10.6.200 | Wireless Access Point — WEP encryption, SSID: BNU-Marketing, VLAN 60 |

Note: Wireless laptops connecting through AP-MKT receive IPs from the VLAN 60 DHCP pool and are on the same network segment as wired staff machines. WEP is the only wireless security in place on AP-MKT.

7.  Network Interconnections

The table below shows all physical links in the BNU campus network as currently deployed.

| From | To | Link Type | Speed | VLAN / Mode |
| --- | --- | --- | --- | --- |
| R-01 Gi0/0 | ISP | Copper Straight | 100 Mbps | WAN (ip nat outside) |
| R-01 Gi0/1 | CS-01 Gi0/1 | GigabitEthernet | 1 Gbps | 802.1Q Trunk — all VLANs |
| CS-01 Fa0/1 | SW-ADM (Admission) | FastEthernet | 100 Mbps | 802.1Q Trunk — VLAN 10 |
| CS-01 Fa0/2 | SW-EXAM (Exam) | FastEthernet | 100 Mbps | 802.1Q Trunk — VLAN 20 |
| CS-01 Fa0/3 | SW-ADMIN (Admin) | FastEthernet | 100 Mbps | 802.1Q Trunk — VLAN 40 |
| CS-01 Fa0/4 | SW-FIN (Finance) | FastEthernet | 100 Mbps | 802.1Q Trunk — VLAN 50 |
| CS-01 Fa0/5 | SW-MKT (Marketing) | FastEthernet | 100 Mbps | 802.1Q Trunk — VLAN 60 |
| CS-01 Fa0/6 | SW-SRV (Server Farm) | FastEthernet | 100 Mbps | 802.1Q Trunk — VLAN 70 |
| SW-MKT Fa0/10 | AP-MKT (WiFi AP) | FastEthernet | 100 Mbps | Access port — VLAN 60 |
| SW-SRV Fa0/1–4 | SRV-01 to SRV-04 | FastEthernet | 100 Mbps | Access ports — VLAN 70 |

8.  Student Tasks — Phase I

Read the complete case study in Sections 1–7 before starting. Your submission for Phase I is a single written report in PDF format — no Cisco Packet Tracer file is required at this stage.

Task 0 — Group Formation  (1 Mark)

- Form a group of 3–4 students. Assign the following roles within your group. Each member must have a clearly defined contribution that is reflected in the submitted report.

- Our group consists of 4 members. Roles were assigned based on individual strengths and divided to ensure every section of the report reflects dedicated effort from a named contributor.

| Member | Roll Number | Assigned Role | Contribution |
| --- | --- | --- | --- |
| Muhammad Arez | F2023-872 | Lead Analyst | Studied the case study in full; produced all six department IP scheme tables and the static device summary for Task 1. |
| Sheikh Muhammad Ibrahim | F2023-630 | Services & Protocol Analyst | Researched and wrote the network services analysis, DHCP relay explanation, and TCP/UDP justification for Task 2. |
| Saad Mughal | F2023-009 | Fault & Risk Analyst | Identified all critical devices and links, authored the scenario response and SPOF analysis for Task 3. |
| Muhammad Ismail Rana | F2023-551 | Recommendations & Compiler | Produced the constraints table for Task 4, all recommendations and justification paragraphs for Task 5, and compiled the final report. |

Task 1 — Existing Network Study & IP Scheme Documentation (2 Marks | CLO-2)

- Study the network described in Sections 1–7 and produce a complete, organised summary of the IP addressing scheme for all six departments.

- For every department, create a table containing: Network address, Subnet mask, Usable host range, Default gateway, DHCP pool range, and a list of any statically assigned devices with their hostnames and IPs.

- Clearly state which devices use DHCP and which use static IPs, and explain why each category is configured the way it is.

- For each department, calculate the total number of usable IP addresses in the assigned subnet and compare it against the actual number of devices. Comment on what you observe.

📋 What to Submit for This Task - A per-department IP scheme table (one table per department — six tables total). - A written explanation (2–3 sentences per department) commenting on the suitability of the subnet size versus the actual device count. - A summary table listing all statically assigned devices across the entire network with hostname, IP, and department.

1.1 Introduction

BNU uses 10.10.0.0/16 split into one /24 VLAN per department. Inter-VLAN routing runs on R-01 via Router-on-a-Stick. The tables below document each department's subnet, DHCP pool, static devices, and utilisation.

1.2 Department IP Scheme Tables

Department 1, Admission (VLAN 10)

| Field | Details |
| --- | --- |
| Network Address | 10.10.1.0 |
| Subnet Mask | 255.255.255.0 (/24) |
| Usable Host Range | 10.10.1.1, 10.10.1.254 |
| Total Usable Hosts | 254 |
| Default Gateway | 10.10.1.1 (R-01 Gi0/1.10) |
| DHCP Pool Range | 10.10.1.100, 10.10.1.200 |
| DNS Server | 10.10.7.10 (SRV-01) |
| Access Switch | SW-ADM, Cisco 2960-24TT |

Device List, VLAN 10:

| Hostname | IP Address | Assignment Method | Role |
| --- | --- | --- | --- |
| ADM-PC01, ADM-PC08 | DHCP (.101, .108) | Dynamic | Admission officer workstations |
| ADM-PC09, ADM-PC12 | DHCP (.109, .112) | Dynamic | Reception terminals (public-facing) |
| ADM-LT01 | DHCP (.130) | Dynamic | Admission manager laptop |
| ADM-SRV01 | 10.10.1.10 | Static | Local file server, admission forms |

Subnet Analysis: /24 gives 254 usable addresses; 13 devices = ~5% utilisation, heavily over-provisioned. ADM-SRV01 is static (service host); reception PCs (ADM-PC09-12) share the VLAN with staff, creating an inter-VLAN security concern.

Department 2, Exam (VLAN 20)

| Field | Details |
| --- | --- |
| Network Address | 10.10.2.0 |
| Subnet Mask | 255.255.255.0 (/24) |
| Usable Host Range | 10.10.2.1, 10.10.2.254 |
| Total Usable Hosts | 254 |
| Default Gateway | 10.10.2.1 (R-01 Gi0/1.20) |
| DHCP Pool Range | 10.10.2.100, 10.10.2.200 |
| DNS Server | 10.10.7.10 (SRV-01) |
| Access Switch | SW-EXAM, Cisco 2960-24TT |

Device List, VLAN 20:

| Hostname | IP Address | Assignment Method | Role |
| --- | --- | --- | --- |
| EXAM-PC01, EXAM-PC10 | DHCP (.101, .110) | Dynamic | Exam administration workstations |
| EXAM-PC11, EXAM-PC15 | DHCP (.111, .115) | Dynamic | Result processing terminals |
| EXAM-SRV01 | DHCP (.150) | Dynamic | Exam database server |
| EXAM-SRV02 | DHCP (.151) | Dynamic | Secondary exam server |

Subnet Analysis: 17 devices on /24 = ~6.7% utilisation. EXAM-SRV01/02 receive DHCP addresses, risky for a server tied to a hostname (exam-db.bnu.edu.pk); a lease change can break the exam DB. Should be static or DHCP-reserved.

Department 3, Admin (VLAN 40)

| Field | Details |
| --- | --- |
| Network Address | 10.10.4.0 |
| Subnet Mask | 255.255.255.0 (/24) |
| Usable Host Range | 10.10.4.1, 10.10.4.254 |
| Total Usable Hosts | 254 |
| Default Gateway | 10.10.4.1 (R-01 Gi0/1.40) |
| DHCP Pool Range | 10.10.4.100, 10.10.4.200 |
| DNS Server | 10.10.7.10 (SRV-01) |
| Access Switch | SW-ADMIN, Cisco 2960-24TT |

Device List, VLAN 40:

| Hostname | IP Address | Assignment Method | Role |
| --- | --- | --- | --- |
| ADMIN-PC01, ADMIN-PC10 | DHCP (.101, .110) | Dynamic | Admin and HR staff workstations |
| ADMIN-PC11, ADMIN-PC15 | DHCP (.111, .115) | Dynamic | Facilities and support terminals |
| ADMIN-SRV01 | 10.10.4.10 | Static | File server, HR documents and admin records |

Subnet Analysis: 16 devices on /24 = ~6.3% utilisation. ADMIN-SRV01 is static as a shared file server. No ACLs means staff from any VLAN can reach it through R-01, a confidentiality risk.

Department 4, Finance (VLAN 50)

| Field | Details |
| --- | --- |
| Network Address | 10.10.5.0 |
| Subnet Mask | 255.255.255.0 (/24) |
| Usable Host Range | 10.10.5.1, 10.10.5.254 |
| Total Usable Hosts | 254 |
| Default Gateway | 10.10.5.1 (R-01 Gi0/1.50) |
| DHCP Pool Range | 10.10.5.100, 10.10.5.180 |
| DNS Server | 10.10.7.10 (SRV-01) |
| Access Switch | SW-FIN, Cisco 2960-24TT |

Device List, VLAN 50:

| Hostname | IP Address | Assignment Method | Role |
| --- | --- | --- | --- |
| FIN-PC01, FIN-PC08 | DHCP (.101, .108) | Dynamic | Finance staff, payroll and budget |
| FIN-PC09, FIN-PC13 | DHCP (.109, .113) | Dynamic | Accounts processing terminals |

Subnet Analysis: 13 devices on /24 = under 6% utilisation. DHCP pool is narrowed to .100-.180 (81 addresses), still oversized. Finance ERP depends on SRV-02:8080 with no ACL restricting other VLANs.

Department 5, Marketing (VLAN 60)

| Field | Details |
| --- | --- |
| Network Address | 10.10.6.0 |
| Subnet Mask | 255.255.255.0 (/24) |
| Usable Host Range | 10.10.6.1, 10.10.6.254 |
| Total Usable Hosts | 254 |
| Default Gateway | 10.10.6.1 (R-01 Gi0/1.60) |
| DHCP Pool Range | 10.10.6.100, 10.10.6.180 |
| DNS Server | 10.10.7.10 (SRV-01) |
| Access Switch | SW-MKT, Cisco 2960-24TT |

Device List, VLAN 60:

| Hostname | IP Address | Assignment Method | Role |
| --- | --- | --- | --- |
| MKT-PC01, MKT-PC08 | DHCP (.101, .108) | Dynamic | Marketing staff wired workstations |
| MKT-LT01, MKT-LT04 | DHCP via WiFi (.110, .113) | Dynamic | Staff laptops via AP-MKT (wireless) |
| AP-MKT | 10.10.6.200 | Static | Wireless Access Point, SSID: BNU-Marketing |

Subnet Analysis: 13 devices on /24 = under 6% utilisation. AP-MKT is static at .200 outside the pool. Wired and wireless share one VLAN, and WEP on AP-MKT is trivially breakable, the worst security exposure on the network.

Department 6, Server Farm (VLAN 70)

| Field | Details |
| --- | --- |
| Network Address | 10.10.7.0 |
| Subnet Mask | 255.255.255.0 (/24) |
| Usable Host Range | 10.10.7.1, 10.10.7.254 |
| Total Usable Hosts | 254 |
| Default Gateway | 10.10.7.1 (R-01 Gi0/1.70) |
| DHCP Pool Range | None, Static assignments only |
| DNS Server | 10.10.7.10 (SRV-01, self) |
| Access Switch | SW-SRV, Cisco 2960-24TT |

Device List, VLAN 70:

| Hostname | IP Address | Assignment Method | Role |
| --- | --- | --- | --- |
| SRV-01 | 10.10.7.10 | Static | DHCP server (all VLANs) + Primary DNS |
| SRV-02 | 10.10.7.11 | Static | Web / LMS server |
| SRV-03 | 10.10.7.12 | Static | Email server |
| SRV-04 | 10.10.7.13 | Static | Backup server |

Subnet Analysis: 4 devices on /24 = ~1.6% utilisation, the lowest of any VLAN, a /28 would suffice. All servers are static because clients, DHCP relay, and DNS forwarding reference fixed IPs. SRV-01 is the campus SPOF for DHCP and DNS.

1.3 Summary Table, All Statically Assigned Devices

| # | Hostname | IP Address | Subnet | Department | VLAN | Role |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ADM-SRV01 | 10.10.1.10 | 10.10.1.0/24 | Admission | 10 | Local file server |
| 2 | ADMIN-SRV01 | 10.10.4.10 | 10.10.4.0/24 | Admin | 40 | HR/Admin file server |
| 3 | AP-MKT | 10.10.6.200 | 10.10.6.0/24 | Marketing | 60 | Wireless access point |
| 4 | SRV-01 | 10.10.7.10 | 10.10.7.0/24 | Server Farm | 70 | DHCP + Primary DNS |
| 5 | SRV-02 | 10.10.7.11 | 10.10.7.0/24 | Server Farm | 70 | Web / LMS |
| 6 | SRV-03 | 10.10.7.12 | 10.10.7.0/24 | Server Farm | 70 | Email |
| 7 | SRV-04 | 10.10.7.13 | 10.10.7.0/24 | Server Farm | 70 | Backup server |
| 8 | CS-01 (Mgmt) | 10.10.0.1 | 10.10.0.0/24 | Core / Data Centre | , | Core switch management |

Why static vs dynamic? Static IPs are assigned to anything other devices must reach by fixed address: servers (SRV-01 DHCP/DNS, SRV-02 ERP, etc.), the ERP/file/exam DB servers, and the AP management interface. End-user PCs and laptops use DHCP so the user population can change without manual reconfiguration. EXAM-SRV01/02 are an exception, they should be static or reserved.

Task 2 — Network Services Analysis  (1 Mark  |  CLO-1)

- List every network service running in this network: DHCP, DNS, NAT/PAT, VLANs, and Router-on-a-Stick.

- For each service write: (a) where it is configured, (b) what purpose it serves in this network, and (c) any observations about how it is currently set up.

- Explain how DHCP relay works in this network. Why is ip helper-address required? What would happen across all departments if it were removed from even one sub-interface?

- Identify the transport protocol (TCP or UDP) and port number used by DHCP and DNS. Justify your answers.

📋 What to Submit for This Task - A services summary table with columns: Service ¦ Configured On ¦ Purpose ¦ Observation. - A short paragraph (5–8 sentences) explaining how DHCP relay works in this specific network and the consequence of removing it. - A one-paragraph answer on TCP/UDP usage for DHCP and DNS with port numbers and justification.

2.1 Services Summary Table

| Service | Configured On | Purpose | Observation |
| --- | --- | --- | --- |
| DHCP | SRV-01 (10.10.7.10) | Automatically assigns IP address, subnet mask, default gateway, and DNS server to all client devices across VLANs 10, 20, 40, 50, and 60 | Centralised on a single server with no failover. SRV-01 failure halts IP assignment for all five client VLANs simultaneously. EXAM-SRV01 and EXAM-SRV02 also receive DHCP addresses, a misconfiguration for server-class devices. |
| DNS | SRV-01 (10.10.7.10) | Resolves internal hostnames within the bnu.edu.pk zone and forwards unknown queries to Google Public DNS (8.8.8.8) for external resolution | Single point of failure, no secondary DNS configured anywhere on the network. If SRV-01 fails, all internal name resolution and external forwarding stops for the entire campus. |
| NAT / PAT | R-01 GigabitEthernet0/0 | Translates all internal 10.10.0.0/16 addresses to the single public IP 202.150.10.5 using Port Address Translation (overload), enabling internet access for all campus devices | PAT shares one public IP across all ~126 campus users. No inbound port forwarding is configured, so no internal server is directly reachable from the internet. R-01 is itself a single point of failure for all outbound traffic. |
| VLANs | CS-01 (propagated to all access switches) | Logically segments the network into six isolated broadcast domains, one per department, reducing unnecessary broadcast traffic and providing basic organisational separation | No ACLs are applied between VLANs, so segmentation provides broadcast isolation only. Any host can route to any other host through R-01. VLAN 30 is absent, indicating ad-hoc network growth over time. |
| Router-on-a-Stick | R-01 GigabitEthernet0/1 sub-interfaces | Provides inter-VLAN routing through a single physical 802.1Q trunk link using one sub-interface per VLAN, each with its own IP address acting as the default gateway for that VLAN | The single Gi0/1 physical link between R-01 and CS-01 is both a performance bottleneck and a single point of failure. All inter-VLAN traffic, all DHCP relay traffic, and all server-bound traffic share this one link. |
| DHCP Relay | R-01, ip helper-address 10.10.7.10 on all five client sub-interfaces | Converts DHCP broadcast requests from client VLANs into unicast packets addressed to SRV-01, allowing a centralised DHCP server to serve multiple VLANs across routed boundaries | Correctly configured on all five client sub-interfaces (Gi0/1.10, .20, .40, .50, .60). If removed from any single sub-interface, all devices on that VLAN lose the ability to obtain new IP addresses from SRV-01. |

2.2 How DHCP Relay Works in This Network

DHCP requests are broadcasts and routers drop broadcasts, so a VLAN 10 client cannot reach SRV-01 on VLAN 70 directly. ip helper-address 10.10.7.10 on each R-01 sub-interface makes R-01 a DHCP relay: it rewrites the broadcast as a unicast to SRV-01 (recording the ingress sub-interface IP as giaddr so SRV-01 picks the correct pool), then relays the reply back. If the command is removed from any one sub-interface, that VLAN loses DHCP entirely, no new leases, no renewals after expiry, while other VLANs keep working.

2.3 Transport Protocols, DHCP and DNS

DHCP runs on UDP (client port 68 to server port 67). UDP is required because the client has no IP yet, so a TCP handshake is impossible; UDP's connectionless, broadcast-capable model fits discovery. DNS uses UDP port 53 for normal queries (low overhead, single request/response) and falls back to TCP 53 for responses over 512 bytes and zone transfers, where reliability and ordering matter.

Task 3 — Problem Identification & Root Cause Analysis  (2 Marks  |  CLO-2)

- Examine the topology and identify every device or link whose failure would cause a loss of connectivity — either for one department or for the entire campus.

- For each such device or link state: (a) its hostname or link description, (b) exactly what stops working when it fails, (c) how many departments are affected, and (d) why the current design allows this.

- Scenario: One morning, all six departments simultaneously lose internet access, cannot reach each other, DHCP stops issuing addresses, and DNS stops resolving names. Based only on the network described in this document, identify the single most likely cause and explain your reasoning in detail.

- Beyond the scenario above, are there any other devices whose failure would cause a similarly widespread outage? Name and justify each one.

📋 What to Submit for This Task - A table listing every identified critical device or link with columns: Device/Link ¦ What Fails ¦ Departments Affected ¦ Root Cause. - A dedicated written section (one full paragraph minimum) answering the scenario question with step-by-step reasoning. - A short written section naming and justifying any additional network-wide single points of failure you identify.

3.1 Critical Device and Link Failure Table

| # | Device / Link | What Fails When It Fails | Departments Affected | Root Cause of Vulnerability |
| --- | --- | --- | --- | --- |
| 1 | R-01 (Edge Router) | All inter-VLAN routing, all internet access, all DHCP relay, all NAT/PAT, DNS forwarding to 8.8.8.8 | All 6 | Single router performing all Layer 3 functions, no redundant router, no HSRP |
| 2 | CS-01 (Core Switch) | All switching between departments; all access-switch uplinks lose connectivity; R-01 loses path to all VLANs | All 6 | Single-tier star topology with one aggregation point, no redundant core switch |
| 3 | R-01 Gi0/1 ↔ CS-01 Gi0/1 Link | All inter-VLAN traffic, DHCP relay, and internet-bound traffic stop | All 6 | Single physical trunk link carrying all VLAN traffic, no redundant cable or EtherChannel |
| 4 | R-01 Gi0/0 ↔ ISP Link | All internet access and external DNS forwarding via 8.8.8.8 | All 6 | Single ISP connection, no secondary WAN link or failover |
| 5 | SRV-01 (DHCP + DNS Server) | No new DHCP leases issued for any VLAN; all internal DNS resolution stops; external forwarding stops | All 6 | Single server hosting two critical services simultaneously, no failover, no secondary DNS |
| 6 | CS-01 Fa0/1 ↔ SW-ADM Link | Entire Admission department loses all network connectivity | 1, Admission | Single uplink from access switch to core, no redundant cable or path |
| 7 | CS-01 Fa0/2 ↔ SW-EXAM Link | Entire Exam department loses all network connectivity | 1, Exam | Single uplink from access switch to core, no redundant cable or path |
| 8 | CS-01 Fa0/3 ↔ SW-ADMIN Link | Entire Admin department loses all network connectivity | 1, Admin | Single uplink from access switch to core, no redundant cable or path |
| 9 | CS-01 Fa0/4 ↔ SW-FIN Link | Entire Finance department loses all network connectivity | 1, Finance | Single uplink from access switch to core, no redundant cable or path |
| 10 | CS-01 Fa0/5 ↔ SW-MKT Link | Entire Marketing department loses all network connectivity | 1, Marketing | Single uplink from access switch to core, no redundant cable or path |
| 11 | CS-01 Fa0/6 ↔ SW-SRV Link | All four Data Centre servers become unreachable from all VLANs; DHCP, DNS, LMS, and email fail | All 6 | Single uplink connecting the entire server farm to the network |
| 12 | SW-SRV (Server Farm Switch) | All four Data Centre servers become unreachable | All 6 | Single switch aggregating all servers, no redundant switch or dual-homed server connections |

3.2 Scenario Analysis, Simultaneous Campus-Wide Outage

Simultaneous loss of internet, inter-VLAN, DHCP, and DNS across all six VLANs points to R-01. R-01 is the only WAN gateway and the only NAT device (internet drops), the only Layer 3 device doing inter-VLAN routing via Router-on-a-Stick (cross-VLAN traffic drops), and the DHCP relay for every client VLAN (no relay = no DHCP, even though SRV-01 is healthy). DNS appears to fail because clients on other VLANs reach SRV-01 only through R-01. A single R-01 outage therefore reproduces every symptom; no other device causes all four simultaneously.

3.3 Additional Network-Wide Single Points of Failure

CS-01 (Core Switch): the sole aggregation point for all six access switches and R-01's only Layer 2 path to the VLANs. Its failure isolates every access switch, equivalent to losing R-01, a full campus blackout. No redundant core, no parallel path.

SRV-01 (DHCP + DNS): the only DHCP server for five client VLANs and the only DNS server, with no secondary. On failure, existing clients survive until their lease expires; afterwards no device can get an IP, internal DNS halts, and external resolution stops, breaking LMS, email, and web.

SW-SRV (Server Farm Switch): the single switch connecting all four Data Centre servers. A hardware failure removes DHCP, DNS, LMS, email, and backup at once, even with R-01 and CS-01 healthy. No dual-homing, so its loss equals losing all servers.

Task 4 — Constraints & Challenges Report  (2 Marks  |  CLO-3)

- Review the entire network documentation and list every constraint, inefficiency, and concern you can identify.

- Organise your findings under the following six categories: (1) Topology & Redundancy, (2) IP Addressing & Subnetting, (3) Security, (4) Network Services, (5) Hardware & Capacity, (6) Scalability.

- You must identify a minimum of 10 distinct issues. Each issue must be described clearly — a single sentence is not sufficient; explain why it is a problem and what the potential impact is.

📋 What to Submit for This Task - A constraints table with columns: # ¦ Issue ¦ Category ¦ Affected Area ¦ Potential Impact. - At least 10 rows in the table, each with a clear, detailed description in the Issue and Potential Impact columns. - A brief introduction paragraph (3–5 sentences) before the table summarising the overall state of the network.

4.1 Introduction

BNU's network is functional but architecturally fragile, extended incrementally with no unifying design. One router handles all Layer 3, one core switch aggregates everything, and one server hosts DHCP and DNS, with no redundancy anywhere. Inter-VLAN security is essentially absent, address space is heavily over-provisioned, and WEP wireless and an unsegmented reception VLAN add easy attack paths. The issues below are grouped by category.

4.2 Constraints and Issues Table

| # | Issue | Category | Affected Area | Potential Impact |
| --- | --- | --- | --- | --- |
| 1 | Single Core Switch (CS-01), No Redundancy | Topology & Redundancy | Entire campus | CS-01 is the sole aggregation point for all six access switches. Any hardware failure, power loss, or software crash simultaneously isolates every department. There is no redundant core switch, no alternative path, and no failover mechanism. A single component failure causes a complete campus-wide outage until the physical device is repaired or replaced, potentially hours or days. |
| 2 | Single Edge Router (R-01), No Redundancy | Topology & Redundancy | Entire campus | R-01 handles all inter-VLAN routing, NAT, DHCP relay, and WAN connectivity simultaneously. Its failure removes internet access, inter-departmental communication, DHCP address assignment, and DNS resolution in a single event. There is no standby router, no HSRP virtual gateway, and no failover path of any kind. |
| 3 | Single ISP Link, No WAN Redundancy | Topology & Redundancy | Internet access (all departments) | The university has exactly one ISP connection on R-01 Gi0/0. A line fault, ISP outage, or interface failure cuts off internet access for all campus users with no fallback path. No secondary ISP, 4G/LTE backup, or policy-based routing exists to maintain connectivity during a WAN failure. |
| 4 | Single Trunk Link Between R-01 and CS-01 | Topology & Redundancy | All inter-VLAN traffic | All VLAN traffic, DHCP relay traffic, and internet-bound flows share a single GigabitEthernet link. This is simultaneously a single point of failure and a bandwidth bottleneck. If the cable, port, or SFP fails, all routing is lost instantly. There is no EtherChannel bundle and no redundant physical cable between these two critical devices. |
| 5 | No ACLs Between VLANs | Security | All departments | No Access Control Lists are applied on any R-01 sub-interface or CS-01 SVI. Any host on any VLAN can freely access any server or workstation on any other VLAN through R-01. A reception terminal used by visiting students can reach Finance workstations, ADMIN-SRV01 (HR records), and all Data Centre servers without restriction. This violates the principle of least privilege and creates a serious data protection risk. |
| 6 | WEP Encryption on AP-MKT | Security | Marketing VLAN 60 | The Marketing department's wireless network uses WEP (Wired Equivalent Privacy), a cryptographic protocol that has been demonstrably broken since 2001. WEP encryption can be cracked passively in under two minutes using freely available tools. Any attacker within radio range can decrypt all wireless traffic, capture credentials, and gain full unauthenticated access to VLAN 60 and, through R-01, to all other VLANs on the campus network. |
| 7 | Student-Facing PCs on Same VLAN as Staff and File Server | Security | Admission VLAN 10 | Reception terminals ADM-PC09 to ADM-PC12 are used by visiting students and members of the public. They reside on VLAN 10 alongside staff workstations and ADM-SRV01, the local file server containing admission forms and documents. A student or attacker seated at a reception terminal can attempt direct access to ADM-SRV01 and scan all other VLAN 10 devices without crossing any security boundary. |
| 8 | Exam Servers Using DHCP Instead of Static IPs | Network Services | Exam VLAN 20 | EXAM-SRV01 and EXAM-SRV02 receive their IP addresses dynamically from the VLAN 20 DHCP pool. Servers providing critical services, exam scheduling software, result databases, must have fixed addresses. If a lease expires and is renewed with a different IP, or if the pool is exhausted, DNS records and application connection strings will point to the wrong address, causing exam software failures potentially during live examination periods. |
| 9 | Single DHCP and DNS Server (SRV-01), No Failover | Network Services | Entire campus | SRV-01 simultaneously runs DHCP for all five client VLANs and DNS for the entire campus, with no secondary server configured for either service. A hardware failure or OS crash on SRV-01 means no new IP addresses can be issued for any VLAN, no internal names can be resolved, and no external queries can be forwarded to 8.8.8.8. The 8-day lease partially delays DHCP impact for existing clients but provides no protection for new connections, rebooted devices, or DNS-dependent services. |
| 10 | Severe IP Address Wastage, Uniform /24 for All Departments | IP Addressing & Subnetting | All VLANs | Every department receives a /24 subnet providing 254 usable addresses, regardless of its actual host count. The largest department has 30 users; the smallest has 4 servers. Across six VLANs this represents over 1,500 permanently wasted addresses. The Server Farm subnet is 1.6% utilised. VLSM should be applied to right-size each subnet and free address space for structured future expansion. |
| 11 | VLAN Numbering Gap, VLAN 30 Missing | IP Addressing & Subnetting | Network documentation | VLANs jump from 20 (Exam) to 40 (Admin), skipping VLAN 30, and the IP plan correspondingly skips 10.10.3.0/24. This gap is evidence of ad-hoc growth, a department or service was likely added and removed without a design review. It creates confusion in documentation, makes VLAN-to-subnet mapping non-intuitive for new IT staff, and signals the absence of any structured IP addressing policy. |
| 12 | No Redundancy for Access Switch Uplinks | Topology & Redundancy | Individual departments | Each of the six departmental access switches connects to CS-01 via a single FastEthernet cable. A broken connector, failed port, or damaged cable instantly isolates the entire department with no automatic recovery. Dual uplinks with Spanning Tree Protocol would allow automatic failover in under two seconds, but no such redundancy is currently deployed. |
| 13 | Router-on-a-Stick Bandwidth Bottleneck | Hardware & Capacity | All inter-VLAN traffic | All inter-VLAN traffic between any two departments must traverse R-01's single GigabitEthernet 0/1 interface, shared across all six VLANs simultaneously. While GigabitEthernet provides 1 Gbps of theoretical throughput, all concurrent inter-VLAN flows compete for this single link. As the campus grows and traffic increases, this architecture becomes a hard performance ceiling that cannot be addressed without a fundamental topology redesign. |
| 14 | No Network Monitoring or Management Infrastructure | Hardware & Capacity | Entire campus | The network operates with no SNMP, syslog, NetFlow, or any other monitoring system. The IT team has no visibility into link utilisation, interface error rates, DHCP pool exhaustion, or security anomalies. Failures are discovered only when users report problems, inflating mean time to detect (MTTD) and mean time to repair (MTTR) unnecessarily. Without monitoring, recurring issues go unidentified and capacity planning is impossible. |
| 15 | Flat Single-Tier Architecture, No Scalability | Scalability | Entire campus | The star topology with one core switch and one router cannot be extended without replacing the core device. CS-01 has 24 FastEthernet ports, with six already used for department uplinks, capacity is limited. There is no distribution layer, no modular design zone, and no documented expansion plan. Adding a new department, a new building, or a new service requires a topology redesign rather than a structured addition to an existing hierarchy. |

Task 5 — Proposed Solution & Improvement Recommendations  (2 Marks  |  CLO-3)

- Based on your findings in Tasks 3 and 4, propose a complete improved network design for BNU.

- For every proposed change write: (a) what you are changing or adding, (b) why it is needed, (c) which Cisco Packet Tracer device you would use to implement it, and (d) the expected benefit.

- Your proposed design must address at minimum: single points of failure and redundancy, IP addressing efficiency using VLSM, inter-VLAN access control, wireless security improvement, DHCP and DNS resilience, and any other issues you identified in Task 4.

📋 What to Submit for This Task - A recommendations table with columns: Change / Addition ¦ Reason ¦ Cisco PT Device ¦ Expected Benefit. - A written justification section (one paragraph per major change) explaining your design decisions. - Note: You are NOT required to build or configure this network in Cisco Packet Tracer for Phase I. Implementation will be required in Phase II.

5.1 Introduction

The recommendations below address every issue from Task 4. The proposal moves BNU from a single-tier, single-router design to a resilient, secure, and scalable network. Core changes: a redundant core pair (CS-01/CS-02) doing Layer 3 SVI routing in place of Router-on-a-Stick; dual edge routers (R-01/R-02) with HSRP and a secondary ISP; deny-by-default inter-VLAN ACLs; WPA2-AES on AP-MKT with a separate Guest VLAN for reception; a secondary DHCP/DNS server (SRV-05) in split-scope with SRV-01; VLSM right-sizing all subnets; dual access-switch uplinks with Rapid-PVST+; and SNMP/syslog monitoring on SRV-06.

5.2 Recommendations Table

| # | Change / Addition | Reason | Cisco PT Device | Expected Benefit |
| --- | --- | --- | --- | --- |
| 1 | Add a second core switch (CS-02) in parallel with CS-01, with dual uplinks from all access switches using Rapid-PVST+ | CS-01 is a campus-wide single point of failure; any failure causes complete outage | Cisco 3560-24PS (second unit) | Eliminates core switch as SPOF; STP ensures loop-free redundancy with automatic sub-second failover |
| 2 | Add a second edge router (R-02) with a secondary ISP link, configured with HSRP on the LAN side | R-01 is the sole gateway for all routing, NAT, and WAN, its failure is a total outage | Cisco 2911 (second unit) | Eliminates router as SPOF; HSRP provides automatic gateway failover transparent to end devices |
| 3 | Enable Layer 3 routing on CS-01 and CS-02 via SVIs; remove Router-on-a-Stick from R-01 | The single R-01 Gi0/1 link is a bottleneck and SPOF for all inter-VLAN traffic | Cisco 3560-24PS (L3 capable) | Inter-VLAN routing at wire speed across the core; R-01 focuses solely on WAN and NAT |
| 4 | Apply extended ACLs on each VLAN SVI to enforce least-privilege inter-VLAN access | No ACLs exist, any host can reach any other, including Finance and HR servers | 3560-24PS SVIs / R-01 extended ACLs | Finance, HR, and server VLANs protected from unauthorised cross-VLAN access |
| 5 | Create a separate Guest VLAN (VLAN 80) for ADM-PC09, PC12 reception terminals | Public-facing terminals on the same VLAN as staff PCs and ADM-SRV01 is a direct security risk | Cisco 2960-24TT (existing SW-ADM) | Visiting users cannot reach staff workstations or the local file server; internet-only access via NAT |
| 6 | Replace WEP with WPA2-AES on AP-MKT | WEP has been cryptographically broken since 2001 and can be cracked in under 2 minutes | Cisco Access Point (PT wireless model) | Wireless traffic protected with AES-128; prevents eavesdropping and unauthorised association |
| 7 | Assign static IPs (or DHCP reservations) to EXAM-SRV01 and EXAM-SRV02 | Servers must have fixed addresses; DHCP leases risk address changes that break application connectivity | Config change on SW-EXAM / SRV-01 DHCP reservation | Exam software always resolves to the correct stable IP; no risk of failure during live exam periods |
| 8 | Deploy a secondary DHCP/DNS server (SRV-05) in the Data Centre | SRV-01 runs both services with no redundancy, its failure disables DHCP and DNS campus-wide | Server-PT (new SRV-05) | If SRV-01 fails, SRV-05 maintains IP assignment and name resolution; eliminates dual-service SPOF |
| 9 | Implement VLSM, resize all subnets to match actual department host counts | All VLANs use /24 despite having 4, 30 devices, wasting over 1,500 IP addresses across the network | Config change only, no new hardware | Efficient address space utilisation; clean documentation; room for structured growth; smaller broadcast domains |
| 10 | Add dual uplinks from each access switch to both CS-01 and CS-02 with Rapid-PVST+ | Single access switch uplinks mean any cable or port failure isolates the entire department | Existing 2960-24TT switches + additional cables | Single link failures trigger automatic STP failover; no departmental outage from physical layer faults |
| 11 | Deploy an SNMP/syslog management server (SRV-06) on a dedicated management VLAN | No monitoring exists, failures are detected only through user complaints, maximising MTTD and MTTR | Server-PT (new SRV-06) | IT team gains real-time visibility into device health, link utilisation, DHCP pool status, and security events |

5.3 Written Justification

Redundant Core Switching and Layer 3 SVIs (Changes 1 & 3)

Replacing Router-on-a-Stick with Layer 3 SVIs on the core is the single biggest structural gain: the 3560 routes between VLANs at hardware speed, removing the R-01 Gi0/1 bottleneck. Adding CS-02 in parallel with CS-01 and dual-homing every access switch under Rapid-PVST+ removes the core SPOF, with sub-two-second failover.

Dual Edge Routers with HSRP (Change 2)

A second Cisco 2911 (R-02) with a secondary ISP and HSRP on the LAN side delivers gateway and WAN redundancy. HSRP exposes a virtual gateway IP shared by R-01 (active) and R-02 (standby), so clients never need reconfiguration on failover. A floating static route on R-02 takes over WAN traffic if the primary ISP drops.

VLAN Access Control Lists (Changes 4 & 5)

Missing inter-VLAN ACLs are the worst security gap. A deny-by-default model is proposed: Finance permitted only to SRV-02:8080, Server Farm accepting only DHCP/DNS/HTTP(S), and Admission split so staff retain internal access while reception ports move to a Guest VLAN (VLAN 80) with internet-only access. ACLs are applied inbound on the relevant SVIs.

Wireless Security Upgrade (Change 6)

Upgrading AP-MKT from WEP to WPA2-AES is configuration-only. WPA2 with AES-CCMP resists the passive sniffing attacks that trivially break WEP's RC4. The AP's security mode and passphrase change and MKT-LT01-LT04 are reconfigured with the new credentials. Guest WiFi should later move to its own SSID/VLAN.

DHCP and DNS Resilience (Changes 7 & 8)

Making EXAM-SRV01/02 static (or DHCP-reserved by MAC) is a zero-cost fix that prevents address drift breaking exam software. Adding SRV-05 as a secondary DHCP/DNS removes the SRV-01 SPOF; a split-scope DHCP (SRV-01 lower half, SRV-05 upper half) keeps leases flowing if either server fails, and DNS resolution survives a single-server outage.

VLSM Redesign (Change 9)

Every VLAN currently has a /24 regardless of size, under 7% utilisation. VLSM right-sizes each subnet: Admission /26, Exam /26, Admin /26, Finance /26, Marketing /27, Server Farm /28. No new hardware, only R-01 sub-interface IPs and SRV-01 pools change, freeing address space for future growth.

Dual Access-Switch Uplinks with STP (Change 10)

Each access switch currently uses a single uplink to CS-01, one cut cable isolates the department. Adding a second uplink to CS-02 with Rapid-PVST+ gives one forwarding and one blocking link, with sub-two-second failover, removing the last department-level SPOF.

SNMP/Syslog Network Monitoring (Change 11)

BNU has no monitoring, alerting, or logging today, failures are only discovered when users complain. SRV-06 on a dedicated management VLAN (VLAN 99) receives SNMP traps from every device and syslog from R-01, R-02, CS-01, CS-02. Operators see link, CPU, and authentication events in near real time, cutting MTTD and MTTR dramatically.

Submission Guidelines

| Item | Requirement |
| --- | --- |
| Submission Format | Single PDF report — uploaded to LMS |
| Report Length | Minimum 10 pages, maximum 15 pages (excluding cover page and references) |
| File Naming | BNU_CN_Phase1_GroupXX_RollNo.pdf |
| Deadline | Sunday, 17 May 2025 — 11:59 PM (no extensions, no late submissions) |
| Cisco Packet Tracer | No .pkt file required for Phase I. Packet Tracer implementation is a Phase II requirement. |
| References | Cite all external sources in APA format at the end of the report |
| Plagiarism | Zero tolerance — any detected plagiarism results in zero marks for the entire group |

Academic Integrity Declaration

By submitting this report, all group members confirm that the work is entirely their own, properly referenced, and that every member has actively contributed to its completion.

| Member Name | Roll Number | Role |
| --- | --- | --- |
| Muhammad Arez | (F2023-872) | Lead Analyst, Network Study & IP Documentation |
| Sheikh Muhammad Ibrahim | (F2023-630) | Services & Protocol Analysis |
| Saad Mughal | (F2023-009) | Problem Identification & Root Cause Analysis |
| Muhammad Ismail Rana | (F2023-551) | Constraints, Recommendations & Report Compilation |

References

Cisco Systems. (2023). Cisco IOS IP addressing services configuration guide. https://www.cisco.com/c/en/us/td/docs/ios-xml/ios/ipaddr/configuration/xe-16/ipaddr-xe-16-book.html

Cisco Systems. (2023). Configuring router-on-a-stick inter-VLAN routing. https://www.cisco.com/c/en/us/support/docs/lan-switching/inter-vlan-routing/41860-howto-L3-intervlanrouting.html

Droms, R. (1997). Dynamic Host Configuration Protocol (RFC 2131). Internet Engineering Task Force. https://datatracker.ietf.org/doc/html/rfc2131

Mockapetris, P. (1987). Domain names, implementation and specification (RFC 1035). Internet Engineering Task Force. https://datatracker.ietf.org/doc/html/rfc1035

Forouzan, B. A. (2021). Data communications and networking (6th ed.). McGraw-Hill Education.

Tanenbaum, A. S., & Wetherall, D. J. (2021). Computer networks (6th ed.). Pearson.

Beaconhouse National University  |  Lab209 – Computer Networks  |  Good Luck!