Name : ________________________________________ Reg. No. :_____________________ Section No: A

Question – 1 												[CLO2]

You are given an IP address: 150.1.2.3.  Find the Class, default subnet mask, number of usable hosts per network. Can you efficiently use it for 600 hosts? Justify your answer.				[3,2]

- Class: B (150 → 128–191)

- Default Mask: 255.255.0.0

- Hosts: 65,534   (65536 – 2 = 65534)								[3]

- Efficiency: Not efficient because only 600 needed, but 65K available → Lots of wastage	[2]

Question – 2												[CLO2]

You are given the network: 192.168.40.0/24. Divide this network into at least 5 equal-sized subnets. Each subnet must support at least 25 hosts. All subnets must be equal in size				[2,6,2]]

- Find the new subnet mask (in binary) and clearly show the working on borrowing bits.

- List the first 5 subnets, and for first five subnets provide:

  - Network address

  - First usable IP

  - Last usable IP

  - Broadcast address

- Identify one inefficiency in your design

Borrow 3 bits from the host part

## New Subnet Mask calculation: Original: /24 and New network block : /27

New Subnet mask =11111111.11111111.11111111.11100000 255.255.255.224	[2]

2 marks for showing binary working of borrowing and calculating the new /custom subnet mask

Subnets = (2^3 = 8 possible subnets)

Host bits left = 5

Total possible host addresses = (2^5 = 32)
Usable host addresses= 32 - 2 = 30 hosts

Meets requirement (≥25 hosts)

## Subnet Calculations (given : 192.168.40.0/24)							[5]

Subnet size = 32											[1] if worked out correctly. Student may not mention clearly but working is correct

### Subnet 1

- Network: 192.168.40.0

- Usable: 192.168.40.1 – 192.168.40.30

- Broadcast: 192.168.40.31

### Subnet 2

- Network: 192.168.40.32

- Usable: 192.168.40.33 – 192.168.40.62

- Broadcast: 192.168.40.63

### Subnet 3

- Network: 192.168.40.64

- Usable: 192.168.40.65 – 192.168.40.94

- Broadcast: 192.168.40.95

### Subnet 4

- Network: 192.168.40.96

- Usable: 192.168.40.97 – 192.168.40.126

- Broadcast: 192.168.40.127

### Subnet 5

- Network: 192.168.40.128

- Usable: 192.168.40.129 – 192.168.40.158

- Broadcast: 192.168.40.159

## This is inefficient because we only needed 5 subnets, but created 8				[2]

- 3 subnets unused  address wastage