Name : ________________________________________ Reg. No. :_____________________ Section No: A

Question – 1 												[CLO2]

Given the default mask: 255.255.255.0 for an IP address, Identify the class and number of total hosts. Use an IP address as an example and clearly identify the network as well as broadcast address. 														[2,3]

The default subnet mask 255.255.255.0 Class C IP addresses.

Number of Total Hosts : 8 bits for hosts = (2^8 = 256)

Excluding Network Address and broadcast address: Usable hosts = 256 − 2 = 254	[2]

TWO examples given below					[3 marks for a correct example]

IP Address: 192.168.1.10

Class: C

Default Mask: 255.255.255.0 (/24)

First 24 bits (192.168.1) → Network part

Last 8 bits (.10) → Host part

Network Address: 192.168.1.0   (set all host bits to 0)

Broadcast Address: 192.168.1.255  (set all host bits to 1)

Usable Hosts: 192.168.1.1 → 192.168.1.254 (254 hosts)   optional to write

IP Address: 192.168.10.20

Class: C

Default Mask: 255.255.255.0 (/24)

First 24 bits (192.168.1) → Network part

Last 8 bits (.10) → Host part

Network Address: 192.168.10.0   (set all host bits to 0)

Broadcast Address: 192.168.10.255  (set all host bits to 1)

Usable Hosts: 192.168.10.1 → 192.168.10.254 (254 hosts)  optional to write

Question – 2												[CLO2]

You are given the network:  192.168.10.0/24. Divide this network into at least 5 subnets.	[1,1,1,5,2]

- What will be the new subnet mask?

- How many subnets will you actually get?

- How many usable hosts per subnet?

- List all subnet addresses

- Considering the requirements, do you see any problem with this division into subnets?

Network: 192.168.10.0/24

To create >= 5 subnets: we need to borrow bits from host portion borrow 3 bits

(2^n >= 5)

(2^3 = 8) = total of 8 possible subnets							[1]

New/Custom subnet mask : /27  							[1]

11111111.111111111.11111111.11100000

255.255.255.224

Remaining host bits = (32 – 27  = 5)

Total hosts per subnet = (2^5 = 32)

Usable hosts = (32 - 2 = 30)								[1]

Block size = 256 − 224 = 32  OR possible host addresses = 2^5 =  32

Subnets: (given: 192.168.10.0)			need first 5 subnet adresses	[5]

- 192.168.10.0/27

- 192.168.10.32/27

- 192.168.10.64/27

- 192.168.10.96/27

- 192.168.10.128/27

- 192.168.10.160/27

- 192.168.10.192/27

- 192.168.10.224/27

Inefficient sub netting because 3 subnets are unused → wastage of address space	[2]