Name : ________________________________________ Reg. No. :_____________________ Section No: A

Question – 1 												[CLO2]

A student was asked to divide the network given as 192.168.60.0/24 into at least 5 equal-sized subnets, each supporting at least 25 hosts. The student prepared the solution given below: 															[3,2,10]

/8   Class A

/16  Class B

/24  Class C		Host : 8 bits

/10  Classless

- New subnet mask: /26     192.168.60. 00000000  22  = 4 subnets  INCORRECT

- Hosts per subnet = 62

Subnets:

- Subnet 1:
Network: 192.168.60.0
Usable: 192.168.60.1 – 192.168.60.62
Broadcast: 192.168.60.63

- Subnet 2:
Network: 192.168.60.64
Usable: 192.168.60.65 – 192.168.60.126
Broadcast: 192.168.60.127

- Subnet 3:
Network: 192.168.60.128
Usable: 192.168.60.129 – 192.168.60.190
Broadcast: 192.168.60.191

- Subnet 4:
Network: 192.168.60.192
Usable: 192.168.60.193 – 192.168.60.254
Broadcast: 192.168.60.255

- Identify all mistakes in the solution

- Provide a brief reason (one sentence) for considering it a mistake

- Provide the correct solution

Comments on Solution

- Student borrowed 2 bits → 4 subnets

- Requirement = at least 5 subnets

Problem/Error: 4 < 5 (requirement not met)

- Hosts requirement (62 ≥ 25) is fulfilled But number of required subnets is NOT met

- /26 gives only 4 subnets

- Need at least 5 → must borrow 3 bits   : 23  = 8 subnets

- /24 + 3 borrowed bits = /27

11111111 . 11111111 . 11111111 . 11100000

255. 255. 255. 224

Correct mask = /27 (255.255.255.224)

## Correct Solution

(2^3 = 8) subnets

New / Custom Subnet mask: /27 → 255.255.255.224

Hosts per Subnet

- (2^5 = 32) → 32 – 2 = 30 usable (≥25)

Correct Subnets (First 5 subnets shown)

Block size/ Subnetwork size = 32 (256 – 224 = 32 OR 2^5 = 32)

### Subnet 1

- Network: 192.168.60.0

- Usable: 192.168.60.1 – 192.168.60.30

- Broadcast: 192.168.60.31

### Subnet 2

- Network: 192.168.60.32

- Usable: 192.168.60.33 – 192.168.60.62

- Broadcast: 192.168.60.63

### Subnet 3

- Network: 192.168.60.64

- Usable: 192.168.60.65 – 192.168.60.94

- Broadcast: 192.168.60.95

### Subnet 4

- Network: 192.168.60.96

- Usable: 192.168.60.97 – 192.168.60.126

- Broadcast: 192.168.60.127

### Subnet 5

- Network: 192.168.60.128

- Usable: 192.168.60.129 – 192.168.60.158

- Broadcast: 192.168.60.159

### Subnet 6 if 6 subnets were required

- Network: 192.168.60.160

- Usable: 192.168.60.161 – 192.168.60.190

- Broadcast: 192.168.60.191