Question – 1 												[CLO3]

A sender wants to transmit the following data using 16-bit words and the 1’s complement checksum method:

7A2C   91F5   3E10   8B77   6D3A

- Compute the checksum to be transmitted.							[4]

- Show the final transmitted message.							[2]

At the receiver end, suppose the received data is:

7A2C   91F5   3E10   8B77   6D8A   along with correct checksum

- Verify whether an error is detected or not.							[5]

- Briefly explain why this error is or is not detected.						[4]

SOLUTION

Final transmitted data: 7A2C   91F5   3E10   8B77  6D3A   BD1B

Yes. the error has been detected. At the receiver end, one word was changed during transmission 6D8A instead of the word (6D3A) sent by the sender. 1-bit change altered the sum

Error has been detected even though a subtle 1-bit error was induced during transmission. So checksum works here and the receiver will know that the data has an error before it is used