Name : ____________________________________________	Reg.. No. _____________________ Section-B

Question – 1 												[CLO3]

Suppose the following 8-bit data words are being transmitted from location-A using Even parity:

1 0 1 1 0 1 1 0

1 1 0 0 1 0 0 1

0 1 1 0 1 1 0 0

1 0 0 1 1 1 0 1

- Show complete working on the sender end using 2-D parity checking				[4]

- Write the finally transmitted data (9-bit words)							[1]

- Suppose two bits got flipped one at row 2 and column 5 and the other at row 3 and column 6 during transmission. Write the received data words at Location-B.					[2]

- Apply 2-D parity checking at the receiver end 							[3]

- List the possible error positions									[2]

- What is expected at the receiver end after detecting error(s) if any?				[3]