BEACONHOUSE NATIONAL UNIVERSITY

LAB 7

Name: Saad Mughal

BNU ID: F2023-009

Total Marks: /10

Due Date: 24 Mar, 2025

Lab Manual

For

COAL(SEC_B)

| Course Instructor | Ms. Nimra Abbas |
| --- | --- |
| Lab Instructor | Ayesha Seher |
| Semester | Spring 2025 |

Problem No.1:

Register Indirect Addressing and array + loop implementation

Your task is to develop an assembly program for an embedded system that processes

sensor readings stored in memory. The system collects 10 sensor values and needs to

compute their total sum for further analysis. The memory location of these sensor values

is stored in an array, and each value is 2 bytes (word) in size. Perform a dry run,

explaining each LOC (Line of code*).

Solution:

org 100h          ; Set the start address for the program

.model small

.data

arr dw 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 ; Array of 9 words

total dw 0                    ; Variable to store the total sum

.code

main proc

; Initialize data segment

mov ax, @data

mov ds, ax

; Initialize sum to 0 and counter to 10

mov ax, 0

mov cx, 10                   ; Set loop counter to 10 (for 10 elements)

lea bx, arr                 ; Load the address of arr into bx

arr1:

add ax, [bx]                ; Add the value at the memory location pointed by bx to ax

add bx, 2                   ; Increment the pointer by 2 (next word in array)

sub cx, 1                   ; Decrement the loop counter (cx)

jnz arr1                    ; Repeat loop if cx is not zero

; Store the result in the total variable

mov [total], ax

; Move the result to dx for display or further usage

mov dx, [total]

; Exit the program

mov ah, 4Ch                 ; Function to exit the program

int 21h                     ; Call DOS interrupt to exit

main endp

end main

Screenshot:

Problem No.2:

Register Indirect Addressing (without loop)

Write a program to add four numbers using Register indirect/ indirect addressing. Also

dry run each LOC.

Solution:

org 100h

.model small

.data

total dw 0

array dw 2, 4, 6, 8

.code

main proc

mov ax, @data

mov ds, ax

mov dx, 0

mov bx, 0

lea bx, array

mov ax, 0

add ax, [bx]

add bx, 2

add ax, [bx]

add bx, 2

add ax, [bx]

add bx, 2

add ax, [bx]

mov [total], ax

mov dx, [total]

mov ah, 4ch

int 21h

main endp

end main

Screenshot:

Problem No.3:

A temperature monitoring system in an industrial plant collects 7 temperature readings

every hour. These readings are stored in a fixed memory location in a word-sized (2-byte)

array.

- Access the stored temperature readings using Register + Offset Addressing.

- Compute the total sum of the 7 readings.

- Store the computed sum in memory for later use by another process.

Scenario Details:

- The sensor readings are stored in a contiguous memory block (num1).

- The array starts at a known memory location.

- The program will traverse the array using BX as an index and accumulate the sum in AX.

- Each reading is 2 bytes, so BX is incremented by 2 in each iteration.

- The program will loop until all 10 readings are processed and store the final sum in total.

Solution:

org 100h

.model small

.data

total dw 0

array dw 2, 4, 6, 8, 10, 12, 14

.code

main proc

mov ax, @data

mov ds, ax

mov ax, 0

mov bx, 0

mov cx, 7

l1:

add ax, [array + bx]  ; register offset which increments by 2

add bx, 2

sub cx, 1

jnz l1                 ; jump until cx is not equals to zero

mov [total], ax       ; ax is saved indirectly to the total

mov dx, [total]

mov ah, 4ch

int 21h

main endp

end main

Screenshot: